ANTENNA Q2342H6XL

DESCRIPTION: 

	Boomlength
	Number of elements
	Diameter of elements 
	Height of elements
	Antenna for very high G/T and broadband characteristic,  
recommended material: aluminum

	3873 mm
	42
	5,0 mm
	38 mm
	


PERFORMANCES:
	Element material
	G (dBi)
	F/B

(dB)
	F/Sh (dBi)
	F/Sv (dBi)
	Hor
(◦)
	Ver
(◦)
	Temp (◦K)
	G/T (dB)
	Bandwidth (MHz)

for SWR=1,5 

	No loss
	21,31
	34,51
	20,61
	20,03
	18,0
	18,2
	10,2
	+11,21
	1275,300-1307,400

	Aluminum rods
	21,27
	34,64
	20,62
	20,05
	18,0
	18,2
	13,0
	+10,13
	

	Silvered rods
	21,28
	34,58
	20,61
	20,04
	18,0
	18,2
	12,0
	+10,49
	


DIMENSIONS:

	
	Ref
	De
	D1
	D2
	D3
	D4
	D5
	D6
	D7
	D8
	D9
	D10
	D11
	D12
	D13
	D14
	D15
	D16

	Pos
	0
	42
	69
	117
	160
	227
	315
	411
	509
	608
	708
	809
	908
	1008
	1108
	1208
	1309
	1413

	Length
	94
	83
	78,5
	70
	69,5
	68,5
	66
	64
	63
	62
	61
	60
	59
	59
	58,5
	58
	57,5
	57

	
	D17
	D18
	D19
	D20
	D21
	D22
	D23
	D24
	D25
	D26
	D27
	D28
	D29
	D30
	D31
	D32
	D33
	D34

	Pos
	1518
	1622
	1729
	1839
	1942
	2050
	2155
	2255
	2351
	2445
	2543
	2643
	2743
	2850
	2950
	3060
	3163
	3265

	Length
	56
	55
	54
	53
	52,5
	51,5
	50,5
	50,5
	50
	50
	50
	50
	50
	49,5
	49,5
	49,5
	49
	49

	
	D35
	D36
	D37
	D38
	D39
	D40
	
	
	
	
	
	
	
	
	
	
	
	

	Pos
	3371
	3474
	3578
	3681
	3792
	3873
	
	
	
	
	
	
	
	
	
	
	
	

	Length
	49
	49
	49
	49
	49
	48,5
	
	
	
	
	
	
	
	
	
	
	
	


	All measures given in milimeters


DIAGRAMS 

NO LOSS INCLUDED
[image: image1.png]Total Field

EZINEC+

Azimuth Plot
Elevation Angle
Outer Fing

3D M Gain
Sice Max Gain
FrortBack
Beamwidth
Sidelobe Gain
FrortiSidelobe

Cursor sz
0.0 deg. Gan
2131 di

2131 di

21,31 dBi @ Az Angle = 0.0 deg.

3451 08
180 deg; 348 @ 3510, 3.0 deg.
07 dBi @ Az Angle = 21 D deg
2081 08

1296.2 MHz

0.0 deg.
2131 di
0.0 dBmex
0.0 dBmaxaD



[image: image2.png]Total Field

EZINEC+

Elevation Piat
azimuth Angle
Outer Fing

3D M Gain
Sice Max Gain
FrortBack
Beamwidth
Sidelobe Gain
FrortiSidelobe

Cursor Elev
0.0 deg. Gan

2131 di

2131 di

21,31 dBi @ Elev Angle = 0.0 deg,
3451 08
182 deg; 348 @ 3509, 31 deg
1.23 85I @ Elev Angle = 21 0 deg.
200308

1296.2 MHz

0.0 deg.
2131 di
0.0 dBmex
0.0 dBmaxaD




LOSSES INCLUDED (aluminum)

[image: image3.png]W

EZNEC+

1275

Freq 129 Mz

SWR 1015

z 43.46 ot 0.5 deg
49,45 -] 0.4759 ohms

Refl Coeff 0.007271 ot 136,55 deg,
= 0005451 -/ 0.004812

Retloss 42808

Freq iz

Source # 1

)

50hms




[image: image4.png]W

EZNEC+

Refl Coeft

RetLoss

12365 MHz
1004
50.14 ot 013 degs

501401101 ohms
0001734 ot -37.75 deg.
0001419000108
54308

Freq iz

1238

Source # 1

)

50hms




[image: image5.png]Total Field

EZINEC+

Azimuth Plot
Elevation Angle
Outer Fing

3D M Gain
Sice Max Gain
FrortBack
Beamwidth
Sidelobe Gain
FrortiSidelobe

Cursor sz
0.0 deg. Gan
2127 di

2127 ai

2127 dBi @ Az Angle = 0.0 deg.

3454 0B
180 deg; 348 @ 351.0,3.0 deg
0.65 481 @ Az Angle = 21.0 ceg.
205208

1296.2 MHz

0.0 deg.
21 27 di
0.0 cBmex
0.0 dBmaxaD



[image: image6.png]Total Field

EZINEC+

Elevation Piat
azimuth Angle
Outer Fing

3D M Gain
Sice Max Gain
FrortBack
Beamwidth
Sidelobe Gain
FrortiSidelobe

Cursor Elev
0.0 deg. Gan

2127 di

2127 ai

21,27 dBi @ Elev Angle = 0.0 deg,
3454 0B
182 de; 348 @ 3509, 31 deg
122 451 @ Elev Angle = 21 0 deg.
200508

1296.2 MHz

0.0 deg.
21 27 di
0.0 cBmex
0.0 dBmaxaD




LOSSES INCLUDED (silver)
[image: image7.png]Total Field

EZINEC+

Azimuth Plot
Elevation Angle
Outer Fing

3D M Gain
Sice Max Gain
FrortBack
Beamwidth
Sidelobe Gain
FrortiSidelobe

Cursor sz
0.0 deg. Gan

21 28 di

212800

2128 o8I @ Az Angle = 0.0 deg.

345808
180 deg; 348 @351.0,3.0 deg
0.67 dBi @ Az Angle = 21.0 deg.
2081 0B

1296.2 MHz

0.0 deg.
21 28 di
0.0 cBmex
0.0 dBmaxaD



[image: image8.png]Total Field

EZINEC+

Elevation Piat
azimuth Angle
Outer Fing

3D M Gain
Sice Max Gain
FrortBack
Beamwidth
Sidelobe Gain
FrortiSidelobe

Cursor Elev
0.0 deg. Gan

21 28 di

212800

21,26 dBi @ Elev Angle = 0.0 deg,
345808
182 deg; 348 @ 3509, 31 deg
1.24 85I @ Elev Angle = 21 0 deg.
20048

1296.2 MHz

0.0 deg.
21 28 di
0.0 cBmex
0.0 dBmaxaD




