ANTENNA Q204H4XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q204H4XL | 4 | 1300 | 10,83 | 15,61 | 25,91 | 17,65 | 55,0 | 60,6 | 307,9 | -14,11 | 3500 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 10,77 | 307,4 | -14,17 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | REMARKS: : - All elements made of Al rods Ǿ5 mm - The height of all rectangles is 550 mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- |
| Pos | 0 | 402 | 769 | 1300 |  |
| Length | 570 | 538 | 506 | 478 |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | BANDWIDTH (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q204H4XL | 1300 | No loss | 10,83 | 307,9 | -14,11 | 142,300-145,800 |
|  |  | Aluminum | 10,77 | 307,4 | -14,17 |  |

DIAGRAMS (no loss included)

DIAGRAMS (losses included)