PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q205D1XL | 5 | 2422 | 12,65 | 19,44 | 20,22 | 12,34 | 43,0 | 47,7 | 282,5 | -11,86 | 1380 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 12,58 | 283,0 | -11,94 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | REMARKS: : - All elements made of Al rods Ǿ5 mm - The height of all rectangles is 350 mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 415 | 1.024 | 1705 | 2422 |  |
| Length | 772 | 690 | 658 | 650 | 630 |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q205D1XL | 2422 | No loss | 12,54 | 277,9 | -11,90 | 143,720-145,100 |
|  |  | Aluminum | 12,58 | 283,0 | -11,94 |  |
| EF0206-5 | 2377 | No loss | 11,91 | 283,0 | -12,61 | 140,300-145,300 |
|  |  | Aluminum | 11,84 | 283,3 | -12,68 |  |

DIAGRAMS (no loss included)

DIAGRAMS (losses included)