ANTENNA Q205G1XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q205G1XL | 5 | 1735 | 11,94 | 18,02 | 24,67 | 15,05 | 47,4 | 52,0 | 289,2 | -12,67 | 3050 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 11,85 | 289,3 | -12,76 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 |  | REMARKS: : - All elements made of Al rods Ǿ5 mm - The height of all rectangles is 470 mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 280 | 526 | 1099 | 1735 |  |  |
| Length | 650 | 606 | 586 | 564 | 556 |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) |
| --- | --- | --- | --- | --- | --- |
| Q206G1XL | 1735 | No loss | 11,94 | 289,2 | -12,67 |
|  |  | Aluminum | 11,85 | 289,3 | -12,76 |
| EF0206-5 | 2377 | No loss | 11,91 | 283,0 | -12,61 |
|  |  | Aluminum | 11,84 | 283,3 | -12,68 |

DIAGRAMS (no loss included)

DIAGRAMS (losses included)