ANTENNA Q208H3XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q208H3XL | 8 | 4705 | 14,43 | 21,50 | 21,69 | 18,48 | 37,4 | 39,2 | 238,7 | -9,35 | 2800 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 14,29 | 240,7 | 9,53 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | REMARKS: - All elements made of Al rods Ǿ3 mm - The height of all rectangles is 500 mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 398 | 778 | 1398 | 2163 | 2985 | 3790 | 4705 |  |
| Length | 612 | 574 | 554 | 540 | 521 | 502 | 492 | 480 |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q208H3XL | 4705 | No loss | 14,43 | 238,7 | -9,35 | 143,100-145,900 |
|  |  | Aluminum | 14,29 | 240,7 | -9,53 |  |
| EF0210-5LT | 5190 | No loss | 14,12 | 224,1 | -9,38 | 142,000-145,790 |
|  |  | Aluminum | 14,01 | 226,0 | -9,53 |  |

DIAGRAMS (no loss included)

LOSSES INCLUDED

STACKING: 4 antennas in H-configuration H=315 cm, V=345 cm

PERFORMANCES:

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| No loss | 20,37 | 241,4 | -346 |
| Aluminum rods | 20,23 | 243,0 | -3,63 |

NO LOSS CONDITION

LOSSES INCLUDED