ANTENNA Q211D8XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q211D8XL | 11 | 6000 | 15,23 | 26,80 | 22,40 | 19,18 | 34,0 | 36,3 | 224,0 | -8,27 | 3710 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 15,14 | 225,5 | -8,39 |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q211D8XL | 6000 | No loss | 15,23 | 224,0 | -8,27 | 141,190-145,700 |
|  |  | Aluminum | 15,14 | 225,5 | -8,39 |  |
| EF0211B-5 | 5980 | No loss | 15,13 | 229,1 | -8,47 | 140,600-145,210 |
|  |  | Aluminum | 15,03 | 230,8 | -8,60 |  |
| EF0211-5 | 6325 | No loss | 15,24 | 226,7 | -8,31 | 142,500-144,750 |
|  |  | Aluminum | 15,14 | 228,5 | -8,45 |  |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | - All elements made of Al rods Ǿ5 mm - The height of all rectangles is 320mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 295 | 532 | 973 | 1395 | 1958 | 2744 | 3608 | 4435 | 5335 | 6000 |  |
| Length | 786 | 748 | 718 | 676 | 668 | 654 | 638 | 620 | 610 | 600 | 590 |  |

DIAGRAMS (no loss included)

LOSSES INCLUDED