ANTENNA Q2338G6XL

DESCRIPTION:

| Boomlength | Number of elements | Diameter of elements | Height of elements | Antenna for high gain and very broadband characteristic, recommended material: aluminum |
| --- | --- | --- | --- | --- |
| 3525 mm | 38 | 4,0 mm | 30 mm |  |

PERFORMANCES:

| Element material | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | Bandwidth (MHz) for SWR=1,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No loss | 21,19 | 45,76 | 21,12 | 20,44 | 18,0 | 18,2 | 10,8 | +10,86 | 1280,800-1311,000 |
| Aluminum rods | 21,14 | 45,40 | 21,12 | 20,43 | 18,0 | 18,2 | 14,0 | +9,68 |  |
| Silvered rods | 21,16 | 45,54 | 21,12 | 20,44 | 18,0 | 18,2 | 12,8 | +10,08 |  |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 | D13 | D14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 37 | 59 | 95 | 161 | 241 | 329 | 421 | 518 | 620 | 721 | 827 | 929 | 1036 | 1138 | 1244 |
| Length | 98,5 | 87,5 | 81,5 | 79 | 76 | 73 | 71 | 70 | 68 | 67 | 66 | 65 | 65 | 64 | 64 | 63 |
|  | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 | D27 | D28 | D29 | D30 |
| Pos | 1353 | 1457 | 1561 | 1665 | 1771 | 1871 | 1977 | 2076 | 2179 | 2283 | 2387 | 2491 | 2597 | 2703 | 2805 | 2918 |
| Length | 62 | 62 | 61 | 61 | 61 | 59 | 58,5 | 57 | 57 | 56 | 56 | 56 | 56 | 55 | 55 | 55 |
|  | D31 | D32 | D33 | D34 | D35 | D36 |  |  |  |  |  |  |  |  |  |  |
| Pos | 3026 | 3127 | 3235 | 3338 | 3442 | 3525 |  |  |  |  |  |  |  |  |  |  |
| Length | 55 | 55 | 55 | 55 | 55 | 54 |  |  |  |  |  |  |  |  |  |  |

All measures given in milimeters

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)