ANTENNA Q2348G6XL

DESCRIPTION:

| Boomlength | Number of elements | Diameter of elements | Height of elements | Antenna for high gain and ultra broadband characteristic, recommended material: aluminum |
| --- | --- | --- | --- | --- |
| 4555mm | 48 | 4,0 mm | 30 mm |  |

PERFORMANCES:

| Element material | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | Bandwidth (MHz) for SWR=1,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No loss | 22,03 | 40,07 | 22,51 | 21,40 | 16,0 | 16,2 | 10,8 | +11,70 | 1280,800-1314,300 |
| Aluminum rods | 21,99 | 40,15 | 22,52 | 21,42 | 16,0 | 16,2 | 13,9 | +10,56 |  |
| Silvered rods | 22,00 | 40,12 | 22,51 | 21,41 | 16,0 | 16,2 | 12,8 | +10,92 |  |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 | D13 | D14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 35 | 57 | 95 | 160 | 240 | 327 | 421 | 518 | 621 | 722 | 827 | 929 | 1036 | 1138 | 1243 |
| Length | 99 | 87,5 | 82 | 79 | 76 | 73 | 70,5 | 69,5 | 67,5 | 66,5 | 66 | 65,5 | 65 | 64,5 | 64,5 | 62,5 |
|  | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 | D27 | D28 | D29 | D30 |
| Pos | 1351 | 1456 | 1562 | 1668 | 1772 | 1870 | 1976 | 2080 | 2181 | 2284 | 2384 | 2489 | 2589 | 2704 | 2802 | 2912 |
| Length | 62 | 61,5 | 61 | 61 | 61 | 60 | 59 | 58 | 57 | 57 | 57 | 57 | 55 | 55 | 55 | 55 |
|  | D31 | D32 | D33 | D34 |  |  |  |  |  |  |  |  |  |  |  |  |
| Pos | 3015 | 3126 | 3227 | 3334 | 3435 | 3538 | 3638 | 3738 | 3845 | 3947 | 4052 | 4158 | 4259 | 4362 | 4475 | 4555 |
| Length | 55 | 55 | 55 | 55 | 55 | 54,5 | 54,5 | 54,5 | 54,5 | 54,5 | 54,5 | 54,5 | 54,5 | 54,5 | 54,5 | 54 |

All measures given in milimeters

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)