ANTENNA Q2354H9XL

DESCRIPTION:

| Boomlength | Number of elements | Diameter of elements | Height of elements | Antenna for very high gain and G/T, recommended material: aluminum |
| --- | --- | --- | --- | --- |
| 5000 mm | 54 | 5,0 mm | 33 mm |  |

PERFORMANCES:

| Element material | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | Bandwidth (MHz) for SWR=1,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No loss | 21,83 | 38,52 | 26,30 | 24,96 | 16,8 | 17,0 | 9,2 | +12,18 | 1283,100-1303,900 |
| Aluminum rods | 21,79 | 38,69 | 26,29 | 24,96 | 16,8 | 17,0 | 12,2 | +10,91 |  |
| Silvered rods | 21,81 | 38,63 | 26,3 | 24,96 | 16,8 | 17,0 | 11,1 | +11.34 |  |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 | D13 | D14 | D15 | D16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 44 | 68 | 111 | 154 | 223 | 308 | 402 | 498 | 601 | 703 | 802 | 902 | 1001 | 1101 | 1196 | 1295 | 1396 |
| Length | 97,5 | 86,5 | 82 | 73,5 | 72,5 | 71,5 | 69,5 | 68 | 66 | 65 | 64,5 | 64 | 63 | 62 | 61 | 60 | 59 | 58,5 |
|  | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 | D27 | D28 | D29 | D30 | D31 | D32 | D33 | D34 |
| Pos | 1495 | 1600 | 1704 | 1809 | 1915 | 2018 | 2120 | 2222 | 2319 | 2419 | 2519 | 2619 | 2719 | 2819 | 2918 | 3017 | 3116 | 3216 |
| Length | 58 | 57,5 | 57 | 56,5 | 56 | 55 | 54,5 | 54 | 53,5 | 53 | 52,5 | 52 | 51,5 | 51 | 50,5 | 50 | 49,5 | 49,5 |
|  | D35 | D36 | D37 | D38 | D39 | D40 | D41 | D42 | D43 | D44 | D45 | D46 | D47 | D48 | D49 | D50 | D51 | D52 |
| Pos | 3315 | 3415 | 3516 | 3616 | 3717 | 3818 | 3918 | 4016 | 4115 | 4215 | 4315 | 4415 | 4515 | 4615 | 4716 | 4816 | 4914 | 5000 |
| Length | 49 | 48,5 | 48,5 | 48,5 | 48,5 | 48 | 48 | 48 | 48 | 47,5 | 47,5 | 47,5 | 47 | 47 | 47 | 47 | 46 | 46 |

All measures given in milimeters

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)