ANTENNA Q712E4XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q712E4XL | 12 | 2645 | 16,17 | 38,39 | 20,34 | 18,13 | 31,2 | 32,4 | 20,5 | +3,04 | 8700 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 16,10 | 25,1 | +2,11 |
| Silvered rods | 16,12 | 23,4 | +2,43 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 104 | 186 | 373 | 623 | 889 | 1195 | 1498 | 1799 | 2109 | 2409 | 2645 |
| Length | 244 | 223 | 214 | 204 | 198 | 191 | 182 | 178 | 176 | 172 | 172 | 170 |
| - All elements made of Al rods Ǿ4,0 mm - The height of all rectangles is 135 mm - All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q712E4XL | 2645 | No loss | 16,17 | 20,5 | +3,04 | 428,300-437,000 |
|  |  | Aluminum | 16,10 | 25,1 | +2,11 |  |
|  |  | Silver | 16,12 | 23,4 | +2,43 |  |
| EF7012-5 | 2417 | No loss | 15,93 | 24,1 | +2,12 | 429,200-434,120 |
|  |  | Aluminum | 15,86 | 28,7 | +1,28 |  |
|  |  | Silver | 15,89 | 27,0 | +1,58 |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)