ANTENNA Q714D7XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q714D7XL | 14 | 3035 | 16,88 | 21,33 | 20,13 | 18,24 | 28,2 | 29,5 | 19,4 | +4,00 | 7600 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 16,84 | 22,3 | +3,35 |
| Silvered rods | 16,86 | 21,2 | +3,59 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 111 | 197 | 352 | 482 | 689 | 938 | 1223 | 1519 | 1825 | 2135 | 2451 | 2754 | 3035 |
| Length | 282 | 258 | 247 | 223 | 221 | 220 | 214 | 208 | 203 | 198 | 194 | 190 | 186 | 182 |
| - All elements made of Al tubes Ǿ8,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q714D7XL | 3035 | No loss | 16,88 | 19,4 | +4,00 | 427,600-435,200 |
|  |  | Aluminum | 16,84 | 22,8 | +2,83 |  |
|  |  | Silver | 16,86 | 21,2 | +3,59 |  |
| EF7014-5 | 3050 | No loss | 16,78 | 22,9 | +3,17 | 430,450-433,650 |
|  |  | Aluminum | 16,70 | 28,4 | +2,16 |  |
|  |  | Silver | 16,73 | 26,4 | +2,52 |  |

DIAGRAMS

NO LOSS INCLUDED)

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)