ANTENNA Q726D5XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q726D5XL | 26 | 6872 | 19,95 | 30,86 | 19,68 | 18,83 | 20,0 | 20,4 | 16,5 | +7,78 | 7500 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 19,90 | 20,4 | +6,80 |
| Silvered rods | 19,92 | 19,0 | +7,14 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 128 | 199 | 350 | 484 | 687 | 939 | 1227 | 1522 | 1825 | 2130 | 2449 | 2767 |
| Length | 282 | 263 | 248 | 229 | 225 | 222 | 215 | 210 | 206 | 202 | 199 | 196 | 193 |
|  | D12 | D13 | D14 | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 |
| Pos | 3089 | 3409 | 3733 | 4062 | 4382 | 4715 | 5045 | 5363 | 5685 | 5996 | 6316 | 6629 | 6872 |
| Length | 191 | 189 | 186 | 184 | 182 | 180 | 180 | 180 | 178 | 176 | 175 | 172 | 170 |
| - All elements made of Al rods Ǿ8,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q726D5XL | 6872 | No loss | 19,95 | 16,5 | +7,78 | 426,900-434,400 |
|  |  | Aluminum | 19,90 | 20,4 | +6,80 |  |
|  |  | Silver | 19,92 | 19,0 | +7,14 |  |
| EF7032-5 | 8670 | No loss | 19,93 | 19,9 | +6,94 | 429,700-433,980 |
|  |  | Aluminum | 19,85 | 25,5 | +5,78 |  |
|  |  | Silver | 19,88 | 23,3 | +6,20 |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)