ANTENNA Q728D2XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q728D2XL | 28 | 7523 | 20,02 | 32,81 | 20,70 | 19,83 | 20,2 | 20,6 | 16,4 | +7,87 | 5100 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 19,95 | 21,1 | +6,70 |
| Silvered rods | 19,98 | 19,4 | +7,10 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 123 | 201 | 348 | 485 | 679 | 936 | 1215 | 1515 | 1824 | 2132 | 2450 | 2770 | 3095 |
| Length | 280 | 255 | 246 | 229 | 224 | 221 | 216 | 211 | 207 | 203 | 200 | 197 | 195 | 193 |
|  | D13 | D14 | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 |
| Pos | 3408 | 3740 | 4072 | 4385 | 4708 | 5036 | 5375 | 5678 | 6012 | 6335 | 6652 | 6930 | 7270 | 7523 |
| Length | 191 | 189 | 187 | 185 | 183 | 181 | 179 | 177 | 175 | 175 | 175 | 172 | 170 | 166 |
| All elements made of Al rods Ǿ5,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q728D2XL | 7523 | No loss | 20,02 | 16,4 | +7,87 | 429,600-434,700 |
|  |  | Aluminum | 19,95 | 21,1 | +6,70 |  |
|  |  | Silver | 19,98 | 19,4 | +7,10 |  |
| EF7032-5 | 8670 | No loss | 19,93 | 19,9 | +6,94 | 429,700-433,980 |
|  |  | Aluminum | 19,85 | 25,5 | +5,78 |  |
|  |  | Silver | 19,88 | 23,3 | +6,20 |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)