ANTENNA Q738D2XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q738D2XL | 38 | 10620 | 20,90 | 38,63 | 22,90 | 22,21 | 18,6 | 18,9 | 16,0 | +8,85 | 5150 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 20,83 | 20,6 | +7,69 |
| Silvered rods | 20,85 | 19,0 | +8.06 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 | D13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 120 | 201 | 346 | 484 | 679 | 936 | 1215 | 1515 | 1824 | 2132 | 2448 | 2769 | 3095 | 3408 |
| Length | 278 | 254 | 245 | 228 | 223 | 220 | 216 | 211 | 207 | 203 | 200 | 197 | 195 | 193 | 191 |
|  | D14 | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 | D27 | D28 |
| Pos | 3740 | 4072 | 4385 | 4708 | 5036 | 5375 | 5678 | 6012 | 6335 | 6653 | 6933 | 7279 | 7565 | 7865 | 8178 |
| Length | 189 | 187 | 185 | 183 | 181 | 179 | 177 | 175 | 175 | 174 | 173 | 173 | 169 | 167 | 165 |
|  | D29 | D30 | D31 | D32 | D33 | D34 | D35 | D36 | - All elements made of Al rods Ǿ5,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |  |
| Pos | 8489 | 8785 | 9119 | 9428 | 9739 | 10048 | 10355 | 10620 |  |  |  |  |  |  |  |
| Length | 165 | 165 | 163 | 162 | 162 | 160 | 158 | 156 |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q738D2XL | 10620 | No loss | 20,90 | 16,0 | +8,85 | 429,450-434,600 |
|  |  | Aluminum | 20,83 | 20,6 | +7,69 |  |
|  |  | Silver | 20,85 | 19,0 | +8.06 |  |
| NO REFERENCE TO COMPARE |  | No loss |  |  |  |  |
|  |  | Aluminum |  |  |  |  |
|  |  | Silver |  |  |  |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)