ANTENNA QY20710XL7D2

Good stacking characteristics at small vertical distances and big horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20710XL7D2 | 7 | 2110 | 11.76 | 28.73 | 24.15 | 20.69 | 48.2 | 57.0 | 279.0 | -12.70 | 8100 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 11.73 | 279.0 | -12.73 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | DQ1 | DQ2 | REMARKS : -All elements made of Al rods Ǿ10 mm - The height of the rectangle is 500 mm - All dimensions given in milimetres |
| Pos | 0 | 190 | 320 | 405 | 695 | 1035 | 1546 | 2110 |  |
| Length | 1018 | 1010 | 950 | 930 | 918 | 874 | 530 | 504 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20710XL7D2 | 2110 | No loss | 11.76 | 279.0 | -12.70 | 138.400- 146.500 |
|  |  | Aluminum | 11.73 | 279.1 | -12.73 |  |
| EF0206-5 | 2377 | No loss | 11.91 | 283.0 | -12.61 | 140.200 – 145.300 |
|  |  | Aluminum | 11.84 | 283.3 | -12.68 |  |
| G0KSC 6ELE EU | 2349 | No loss | 12.01 | 272.1 | -12.34 | 141.100 – 145.750 |
|  |  | Aluminum | 11.96 | 272.3 | -12.39 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20710XL7D2 (including loss for aluminum – AGT = -0.03 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 13.38 | 235.7 | -10.34 | 100 |  |  |  |
| 110 | 13.53 | 231.7 | -10.12 | 110 | 13.05 | 276.3 | -11.36 |
| 120 | 13.67 | 229.2 | -9.93 | 120 | 13.25 | 271.9 | -11.09 |
| 130 | 13.80 | 227.9 | -9.78 | 130 | 13.42 | 269.7 | -10.89 |
| 140 | 13.92 | 227.0 | -9.64 | 140 | 13.57 | 269.0 | -10.73 |
| 150 | 14.03 | 225.7 | -9.51 | 150 | 13.71 | 269.4 | -10.59 |
| 160 | 14.13 | 224.4 | -9.38 | 160 | 13.84 | 270.6 | -10.48 |
| 170 | 14.24 | 224.2 | -9.27 | 170 | 13.96 | 272.4 | -10.39 |
| 180 | 14.34 | 225.2 | -9.19 | 180 | 14.07 | 274.4 | -10.31 |
| 190 | 14.43 | 227.5 | -9.14 | 190 | 14.17 | 275.4 | -10.25 |
| 200 | 14.52 | 230.9 | -9.11 | 200 | 14.27 | 278.4 | -10.18 |
| 210 | 14.60 | 235.9 | -9.13 | 210 | 14.37 | 280.1 | -10.10 |
| 220 | 14.67 | 242.4 | -9.18 | 220 | 14.45 | 281.4 | -10.04 |
| 230 | 14.72 | 250.2 | -9.26 | 230 | 14.52 | 282.5 | -9.99 |
| 240 | 14.76 | 258.8 | -9.37 | 240 | 14.59 | 283.2 | -9.93 |
| 250 | 14.79 | 267.8 | -9.49 | 250 | 14.64 | 283.6 | -9.89 |
| 260 | 14.81 | 276.6 | -9.61 | 260 | 14.69 | 283.7 | -9.84 |
| 270 | 14.82 | 285.0 | -9.73 | 270 | 14.72 | 283.6 | -9.81 |
| 280 |  |  |  | 280 | 14.75 | 283.3 | -9.77 |
| 290 |  |  |  | 290 | 14.77 | 282.9 | -9.75 |
| 300 |  |  |  | 300 | 14.79 | 282.3 | -9.72 |
| 310 |  |  |  | 310 | 14.79 | 282.8 | -9.71 |
| 320 |  |  |  | 320 | 14.80 | 281.2 | -9.69 |
| 330 |  |  |  | 330 | 14.80 | 280.6 | -9.68 |
| 340 |  |  |  | 340 | 14.80 | 280.0 | -9.67 |

FOUR ANTENNAS IN H STACK

Recommended stacking distances: 310 cm horizontally and 190 cm vertically

| Antenna | Stacking distance |  | Material | Gain (dBi) | TA (◦K) | G/T (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Horizontally | Vertically |  |  |  |  |
| QY20705XL7D2 | 310 cm | 190 cm | No loss | 17.57 | 225.6 | -5.96 |
|  |  |  | Aluminum | 17.55 | 226.1 | -5.99 |

X POL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change position of D1 for +6 mm

- Change length of DE2 from 950 to 970 mm

In this case the SWR diagram will be like this:

| XPOL PERFORMANCES |  |  |  |  |
| --- | --- | --- | --- | --- |
| VERTICAL COMPONENT | Material | G (dB) | TA (°K) | G/T (dB) |
|  | No loss | 11.66 | 264.6 | -12.57 |
|  | Aluminum | 11.64 | 264.7 | -12.59 |