ANTENNA QY20805XL4D3

The antenna with extremly big working bandwidth. Good for stacking at small vertical distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20805XL4D3 | 8 | 2785 | 12.58 | 20.68 | 23.01 | 17.17 | 44.6 | 50.8 | 262.0 | -11.60 | 7400 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 12.53 | 262.4 | -11.66 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20805XL443 | 2390 | No loss | 12.58 | 262.0 | -11.60 | 141.000 – 148.400 |
|  |  | Aluminum | 12.53 | 262.4 | -11.66 |  |
| G0KSC 6 ELE EU | 2349 | No loss | 12.01 | 272.1 | -12.34 | 141.100 - 145.750 |
|  |  | Aluminum | 11.96 | 272.3 | -12.39 |  |
| EF0206-5 | 2377 | No loss | 11.91 | 283.0 | -12.61 | 140.200 - 145.300 |
|  |  | Aluminum | 11.84 | 283.3 | -12.68 |  |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | DQ1 | DQ2 | DQ3 | REMARKS: - All elements made of Al rods Ǿ5 mm - Height of all rectangles 500 mm - All dimensions given in milimetres |
| Pos | 0 | 190 | 320 | 414 | 689 | 1055 | 1565 | 2227 | 2785 |  |
| Length | 1024 | 1010 | 870 | 942 | 930 | 910 | 524 | 514 | 498 |  |

Building hints: No boom correction included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20805XL4D3 - STACKING TABLE (including loss for aluminum – AGT = -0.05 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dBi) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 14.05 | 230.3 | -9.57 | 100 |  |  |  |
| 110 | 14.19 | 228.0 | -9.39 | 110 |  |  |  |
| 120 | 14.31 | 228.3 | -9.28 | 120 | 13.95 | 258.4 | -10.17 |
| 130 | 14.41 | 230.7 | -9.22 | 130 | 14.11 | 255.1 | -9.96 |
| 140 | 14.49 | 233.7 | -9.20 | 140 | 14.25 | 253.7 | -9.79 |
| 150 | 14.57 | 235.4 | -9.15 | 150 | 14.37 | 253.7 | -9.67 |
| 160 | 14.66 | 236.1 | -9.07 | 160 | 14.49 | 254.6 | -9.57 |
| 170 | 14.73 | 237.3 | -9.02 | 170 | 14.59 | 256.1 | -9.49 |
| 180 | 14.81 | 238.7 | -8.97 | 180 | 14.69 | 258.0 | -9.43 |
| 190 | 14.89 | 239.5 | -8.90 | 190 | 14.79 | 260.0 | -9.36 |
| 200 | 14.98 | 239.6 | -8.82 | 200 | 14.89 | 261.9 | -9.29 |
| 210 | 15.08 | 239.6 | -8.71 | 210 | 14.98 | 263.7 | -9.23 |
| 220 | 15.19 | 240.0 | -8.61 | 220 | 15.07 | 265.2 | -9.17 |
| 230 | 15.28 | 241.7 | -8.55 | 230 | 15.15 | 266.4 | -9.11 |
| 240 | 15.37 | 244.5 | -8.51 | 240 | 15.23 | 267.3 | -9.04 |
| 250 | 15.44 | 248.4 | -8.51 | 250 | 15.30 | 267.8 | -8.98 |
| 260 | 15.50 | 253.1 | -8.53 | 260 | 15.36 | 268.1 | -8.92 |
| 270 | 15.55 | 258.2 | -8.57 | 270 | 15.42 | 268.1 | -8.86 |
| 280 | 15.59 | 263.4 | -8.62 | 280 | 15.46 | 267.9 | -8.82 |
| 290 |  |  |  | 290 | 15.50 | 267.5 | -8.77 |
| 300 |  |  |  | 300 | 15.53 | 267.0 | -8.74 |
| 310 |  |  |  | 310 | 15.55 | 266.4 | -8.71 |
| 320 |  |  |  | 320 | 15.57 | 265.8 | -8.67 |
| 330 |  |  |  | 330 | 15.58 | 265.1 | -8.65 |
| 340 |  |  |  | 340 | 15.59 | 264.4 | -8.63 |
| 350 |  |  |  | 350 | 15.60 | 263.7 | -8.61 |
| 360 |  |  |  | 360 | 15.60 | 263.1 | -8.60 |

TWO ANTENNAS VERTICALLY STACKED AT 240CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| QY20805XL4D3 | No losss | 15.42 | 21.89 | 26.31 | 12.08 | 45.0 | 22.6 | 243.9 | -8.45 | 141.000–148.400 |
|  | Aluminum | 15.37 | 21.82 | 26.25 | 11.99 | 45.0 | 22.6 | 244.5 | -8.51 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 250+250+250CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20805XL4D3 | No losss | 18.46 | 23.42 | 28.22 | 12.53 | 45.8 | 12.62 | 232.9 | -5.21 | 141.000–148.400 |
|  | Aluminum | 18.42 | 23.35 | 28.18 | 12.63 | 45.6 | 12.63 | 233.6 | -5.26 |  |

FOUR ANTENNAS IN H- STACK, H=340CM, V=240 CM, LOSS INCLUDED FOR ALUMINUM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20805XL4D3 | No losss | 18.50 | 21.56 | 7.13 | 11.89 | 16.2 | 22.6 | 246.3 | -5.42 | 141.000 – 148.400 |
|  | Aluminum | 18.45 | 21.50 | 7.14 | 11.91 | 16.2 | 22.6 | 246.9 | -5.48 |  |

EIGHT ANTENNAS IN H- STACK, H=340 CM, V=240+240+240 CM, LOSS INCLUDED FOR ALUMINUM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 8xQY20805XL4D3 | No losss | 21.42 | 22.57 | 7.02 | 12.52 | 16.2 | 10.8 | 235.7 | -2.3 | 141.000 – 148.400 |
|  | Aluminum | 21.37 | 22.50 | 7.03 | 12.53 | 16.2 | 10.8 | 236.4 | -2.37 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections

If you want to get better SWR diagram for vertical antenna you can do the following:

- To change position of D1 for +6 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| Horizontal part | No losss | 12.58 | 20.68 | 23.01 | 17.17 | 44.6 | 50.8 | 262.0 | -11.60 | 141.000 – 148.400 |
|  | Aluminum | 12.53 | 20.63 | 23.03 | 17.15 | 44.6 | 50.8 | 262.4 | -11.66 |  |
| Vertical part | No losss | 12.52 | 19.34 | 17.13 | 23.63 | 51.0 | 44.8 | 251.6 | -11.49 | 141.100 – 148.000 |
|  | Aluminum | 12.47 | 19.29 | 17.12 | 23.59 | 51.0 | 44.8 | 252.3 | -11.55 |  |