ANTENNA QY20806XL4D1

Good stacking characteristics at moderate vertical distances and any horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20806XL4D1 | 8 | 3455 | 13.41 | 21.10 | 23.70 | 18.02 | 40.2 | 45.8 | 250.0 | -10.57 | 5850 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 13.34 | 250.8 | -10.65 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | REMARKS : -All elements made of Al rods Ǿ6 mm - The height of the rectangle is 500 mm - All dimensions given in milimetres |
| Pos | 0 | 180 | 350 | 418 | 709 | 1269 | 1954 | 2795 | 3455 |  |
| Length | 1014 | 1010 | 914 | 948 | 954 | 926 | 904 | 882 | 522 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20806XL4D1 | 3455 | No loss | 13.41 | 250.0 | -10.57 | 140.200- 146.150 |
|  |  | Aluminum | 13.34 | 250.8 | -10.65 |  |
| EF0207-5 | 3274 | No loss | 12.95 | 255.9 | -11.13 | 140.400- 145.100 |
|  |  | Aluminum | 12.85 | 256.8 | -11.25 |  |
| EF0208-5 | 3880 | No loss | 13.52 | 248.2 | -10.43 | 144.220-145.490 |
|  |  | Aluminum | 13.45 | 249.2 | -10.51 |  |
| G0KSC 8 EURO | 3729 | No loss | 13.35 | 236.5 | -10.39 | 141.600-145.450 |
|  |  | Aluminum | 13.28 | 237.4 | -10.47 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| ANTENNA QY20806XL4D1 (including loss for aluminum – AGT = -0.08 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 14.63 | 233.3 | -9.05 | 100 |  |  |  |
| 110 | 14.78 | 228.3 | -8.81 | 110 |  |  |  |
| 120 | 14.92 | 225.7 | -8.62 | 120 | 14.33 | 266.1 | -9.92 |
| 130 | 15.04 | 225.4 | -8.49 | 130 | 14.61 | 258.4 | -9.51 |
| 140 | 15.15 | 226.0 | -8.39 | 140 | 14.81 | 253.0 | -9.22 |
| 150 | 15.25 | 226.4 | -8.30 | 150 | 14.96 | 249.4 | -9.01 |
| 160 | 15.35 | 226.9 | -8.21 | 160 | 15.10 | 247.2 | -8.83 |
| 170 | 15.44 | 228.6 | -8.15 | 170 | 15.22 | 246.0 | -8.69 |
| 180 | 15.53 | 230.8 | -8.10 | 180 | 15.34 | 245.5 | -8.56 |
| 190 | 15.61 | 232.3 | -8.05 | 190 | 15.44 | 245.7 | -8.46 |
| 200 | 15.71 | 232.5 | -7.96 | 200 | 15.54 | 246.2 | -8.37 |
| 210 | 15.81 | 232.1 | -7.85 | 210 | 15.64 | 247.0 | -8.29 |
| 220 | 15.91 | 231.8 | -7.74 | 220 | 15.73 | 247.9 | -8.21 |
| 230 | 16.00 | 232.3 | -7.66 | 230 | 15.81 | 248.8 | -8.15 |
| 240 | 16.09 | 233.7 | -7.60 | 240 | 15.90 | 249.7 | -8.07 |
| 250 | 16.16 | 236.3 | -7.57 | 250 | 15.97 | 250.5 | -8.02 |
| 260 | 16.23 | 239.7 | -7.57 | 260 | 16.05 | 251.2 | -7.95 |
| 270 | 16.28 | 243.7 | -7.59 | 270 | 16.11 | 251.7 | -7.90 |
| 280 | 16.33 | 248.0 | -7.61 | 280 | 16.17 | 252.2 | -7.85 |
| 290 |  |  |  | 290 | 16.22 | 252.4 | -7.80 |
| 300 |  |  |  | 300 | 16.26 | 252.6 | -7.76 |
| 310 |  |  |  | 310 | 16.29 | 252.7 | -7.74 |
| 320 |  |  |  | 320 | 16.32 | 252.7 | -7.71 |
| 330 |  |  |  | 330 | 16.35 | 252.6 | -7.67 |
| 340 |  |  |  | 340 | 16.36 | 252.6 | -7.66 |
| 350 |  |  |  | 350 | 16.38 | 252.5 | -7.64 |
| 360 |  |  |  | 360 | 16.39 | 252.4 | -7.63 |
| 370 |  |  |  | 370 | 16.39 | 252.3 | -7.63 |

TWO ANTENNAS VERTICALLY STACKED AT 250CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| QY20806XL4D1 | No losss | 16.24 | 22.65 | 26.70 | 13.45 | 40.2 | 21.4 | 235.2 | -7.48 | 140.200- 146.150 |
|  | Aluminum | 16.16 | 22.53 | 26.67 | 13.48 | 40.2 | 21.4 | 236.3 | -7.57 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 250+250+250CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20806XL4D1 | No losss | 19.14 | 23.75 | 29.27 | 13.04 | 40.4 | 10.2 | 226.6 | -4.41 | 140.200- 146.150 |
|  | Aluminum | 19.06 | 23.61 | 29.22 | 13.04 | 40.4 | 10.2 | 227.9 | -4.52 |  |

FOUR ANTENNAS IN H- STACK, H=320 CM, V=230 CM, LOSS INCLUDED FOR ALUMINUM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20806XL4D1 | No losss | 19.13 | 22.73 | 9.95 | 15.97 | 17.0 | 23.0 | 231.0 | -4.51 | 140.200- 146.150 |
|  | Aluminum | 19.05 | 22.60 | 9.97 | 16.01 | 17.0 | 23.0 | 232.1 | -4.61 |  |

X POL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE1 from 1010 to 1014 mm

- Change length of DE2 from 900 to 904 mm

- Change position of D1 for +3 mm

- Change position of D5 for -15 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 13.41 | 21.10 | 23.70 | 18.02 | 40.2 | 45.8 | 250.0 | -10.57 | 140.200 - 146.150 |
|  |  | Aluminum | 13.34 | 21.01 | 23.65 | 18.00 | 40.0 | 45.6 | 250.8 | -10.65 |  |
|  | Vertical part | No losss | 13.54 | 19.65 | 17.85 | 23.72 | 45.8 | 40.2 | 234.6 | -10.16 | 140.200 – 146.050 |
|  |  | Aluminum | 13.46 | 19.57 | 17.82 | 23.67 | 45.8 | 40.2 | 235.7 | -10.26 |  |