ANTENNA QY20810XL5D3

Good stacking characteristics at moderate vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20810XL5D3 | 8 | 2885 | 12.88 | 27.25 | 21.31 | 16.19 | 43.2 | 48.2 | 256.2 | -11.21 | 5750 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 12.85 | 253.5 | -11.20 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | DQ1 | DQ2 | DQ3 | REMARKS: -All elements made of Al rods Ǿ10mm - The height of all rectangles 500 mm - All dimensions given in milimetres |
| Pos | 0 | 185 | 322 | 408 | 719 | 1055 | 1558 | 2276 | 2885 |  |
| Length | 1016 | 1000 | 890 | 926 | 912 | 876 | 538 | 522 | 498 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20810XL5D3 | 2885 | No loss | 12.88 | 256.2 | -11.21 | 140.450- 146.200 |
|  |  | Aluminum | 12.85 | 256.5 | -11.24 |  |
| G0KSC 7ELE EU | 2894 | No loss | 12.89 | 260.5 | -11.27 | 141.300- 145.400 |
|  |  | Aluminum | 12.83 | 260.9 | -11.34 |  |
| EF0207-5 | 3274 | No loss | 12.95 | 255.9 | -11.13 | 140.400-145.100 |
|  |  | Aluminum | 12.85 | 256.8 | -11.25 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20810XL5D3 - STACKING TABLE (including loss for aluminum – AGT = -0.03 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 14.22 | 231.9 | -9.43 | 100 |  |  |  |
| 110 | 14.38 | 227.2 | -9.18 | 110 |  |  |  |
| 120 | 14.52 | 225.0 | -9.00 | 120 | 14.20 | 255.8 | -9.88 |
| 130 | 14.64 | 225.1 | -8.88 | 130 | 14.37 | 251.2 | -9.63 |
| 140 | 14.75 | 226.7 | -8.89 | 140 | 14.51 | 249.1 | -9.45 |
| 150 | 14.84 | 228.6 | -8.75 | 150 | 14.64 | 248.4 | -9.31 |
| 160 | 14.92 | 230.8 | -8.71 | 160 | 14.76 | 248.7 | -9.20 |
| 170 | 15.00 | 233.5 | -8.68 | 170 | 14.87 | 249.7 | -9.10 |
| 180 | 15.07 | 236.2 | -8.66 | 180 | 14.97 | 251.1 | -9.03 |
| 190 | 15.15 | 237.8 | -8.61 | 190 | 15.07 | 252.7 | -8.96 |
| 200 | 15.25 | 238.0 | -8.52 | 200 | 15.17 | 254.4 | -8.89 |
| 210 | 15.35 | 237.5 | -8.41 | 210 | 15.26 | 256.1 | -8.82 |
| 220 | 15.46 | 237.1 | -8.29 | 220 | 15.35 | 257.5 | -8.75 |
| 230 | 15.56 | 237.6 | -8.20 | 230 | 15.43 | 258.7 | -8.70 |
| 240 | 15.65 | 239.1 | -8.14 | 240 | 15.51 | 259.6 | -8.63 |
| 250 | 15.73 | 241.8 | -8.10 | 250 | 15.59 | 260.2 | -8.56 |
| 260 | 15.80 | 245.4 | -8.10 | 260 | 15.65 | 260.6 | -8.51 |
| 270 | 15.85 | 249.7 | -8.12 | 270 | 15.71 | 260.7 | -8.45 |
| 280 |  |  |  | 280 | 15.76 | 260.0 | -8.40 |
| 290 |  |  |  | 290 | 15.80 | 260.4 | -8.36 |
| 300 |  |  |  | 300 | 15.84 | 260.0 | -8.31 |
|  |  |  |  | 310 | 15.86 | 259.5 | -8.28 |
|  |  |  |  | 320 | 15.89 | 258.9 | -8.24 |
|  |  |  |  | 330 | 15.90 | 258.3 | -8.22 |
|  |  |  |  | 340 | 15.91 | 257.8 | -8.20 |
|  |  |  |  | 350 | 15.92 | 257.2 | -8.18 |
|  |  |  |  | 360 | 15.92 | 256.7 | -8.18 |

TWO ANTENNAS VERTICALLY STACKED AT 250CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| QY20810XL5D3 | No losss | 15.76 | 27.62 | 23.92 | 12.09 | 43.4 | 21.6 | 241.4 | -8.07 | 140.450- 146.200 |
|  | Aluminum | 15.73 | 27.62 | 23.88 | 12.10 | 43.4 | 21.6 | 241.8 | -8.10 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 250+250+250CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20810XL5D3 | No losss | 18.70 | 28.65 | 26.01 | 12.62 | 43.8 | 10.2 | 229.6 | -4.91 | 140.450- 146.200 |
|  | Aluminum | 18.67 | 28.64 | 25.98 | 12.62 | 43.8 | 10.2 | 230.1 | -4.95 |  |

FOUR ANTENNAS IN H- STACK, H=350 CM, V=244 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20810XL5D3 | No losss | 18.80 | 27.04 | 7.23 | 12.70 | 15.6 | 22.0 | 240.8 | -5.02 | 140.450- 146.200 |
|  | Aluminum | 18.77 | 27.05 | 7.24 | 12.72 | 15.6 | 22.0 | 241.2 | -5.05 |  |

X POL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE2 from 890 to 906 mm

- Change length of D1 from 926 to 924 mm

- Change position of D1 for +9 mm.

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 12.88 | 27.25 | 21.31 | 16.19 | 43.2 | 48.2 | 256.2 | -11.21 | 140.450- 146.200 |
|  |  | Aluminum | 12.85 | 27.25 | 21.29 | 16.18 | 43.2 | 48.2 | 256.5 | -11.24 |  |
|  | Vertical part | No losss | 12.76 | 22.05 | 16.27 | 22.88 | 49.0 | 43.8 | 250.4 | -11.23 | 140.200 – 147.000 |
|  |  | Aluminum | 12.73 | 22.05 | 16.26 | 22.86 | 49.0 | 43.8 | 250.8 | -11.26 |  |