ANTENNA QY20905XL3D4

The gain of this antenna is a little bit reduced, so that its working bandwidth is significantly increased. Good stacking characteristics at moderate vertical distances and big horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
|  | QY20905XL3D4 | 9 | 3785 | 13.33 | 18.85 | 25.89 | 16.99 | 42.0 | 46.2 | 246.0 | -10.58 | 8300 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 13.28 | 246.7 | -10.64 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20905XL3D4 | 3785 | No loss | 13.33 | 246.0 | -10.58 | 139.800- 148.100 |
|  |  | Aluminum | 13.28 | 246.7 | -10.64 |  |
| G0KSC 8 EURO | 3729 | No loss | 13.35 | 236.5 | -10.39 | 141.600-145.450 |
|  |  | Aluminum | 13.28 | 237.4 | -10.47 |  |
| EF0208-5 | 3880 | No loss | 13.52 | 248.2 | -10.43 | 141.220- 145.490 |
|  |  | Aluminum | 13.45 | 249.2 | -10.51 |  |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | DQ1 | DQ2 | DQ3 | DQ4 | REMARKS: - All elements made of Al rods Ǿ5 mm - Height of all rectangles 500 mm - All dimensions given in milimetres |
| Pos | 0 | 180 | 335 | 417 | 694 | 1065 | 1585 | 2247 | 3035 | 3785 |  |
| Length | 1024 | 1020 | 900 | 940 | 936 | 916 | 526 | 512 | 490 | 470 |  |

Building hints: No boom correction included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20905XL3D4 (including loss for aluminum – AGT = -0.06 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 14.79 | 223.4 | -8.70 | 100 |  |  |  |
| 110 | 14.91 | 221.3 | -8.54 | 110 |  |  |  |
| 120 | 15.00 | 222.1 | -8.47 | 120 | 14.69 | 246.9 | -9.24 |
| 130 | 15.08 | 226.0 | -8.46 | 130 | 14.83 | 243.9 | -9.04 |
| 140 | 15.14 | 230.9 | -8.49 | 140 | 14.94 | 242.3 | -8.90 |
| 150 | 15.20 | 233.6 | -8.48 | 150 | 15.05 | 241.9 | -8.79 |
| 160 | 15.28 | 234.0 | -8.41 | 160 | 15.14 | 242.2 | -8.70 |
| 170 | 15.35 | 234.6 | -8.35 | 170 | 15.23 | 243.2 | -8.63 |
| 180 | 15.42 | 236.4 | -8.32 | 180 | 15.32 | 244.5 | -8.56 |
| 190 | 15.48 | 238.3 | -8.29 | 190 | 15.41 | 245.9 | -8.50 |
| 200 | 15.55 | 239.4 | -8.24 | 200 | 15.50 | 247.4 | -8.43 |
| 210 | 15.63 | 239.7 | -8.17 | 210 | 15.58 | 248.7 | -8.38 |
| 220 | 15.72 | 239.5 | -8.07 | 220 | 15.67 | 249.8 | -8.31 |
| 230 | 15.81 | 239.2 | -7.98 | 230 | 15.75 | 250.7 | -8.24 |
| 240 | 15.90 | 239.3 | -7.89 | 240 | 15.84 | 251.4 | -8.16 |
| 250 | 15.99 | 240.0 | -7.81 | 250 | 15.91 | 251.8 | -8.10 |
| 260 | 16.07 | 241.4 | -7.76 | 260 | 15.99 | 251.9 | -8.02 |
| 270 | 16.14 | 243.4 | -7.72 | 270 | 16.05 | 251.9 | -7.96 |
| 280 | 16.20 | 245.8 | -7.71 | 280 | 16.11 | 251.7 | -7.90 |
| 290 | 16.26 | 248.5 | -7.69 | 290 | 16.16 | 251.4 | -7.84 |
| 300 | 16.30 | 251.3 | -7.70 | 300 | 16.21 | 251.0 | -7.79 |
| 310 | 16.34 | 253.9 | -7.71 | 310 | 16.25 | 250.6 | -7.74 |
| 320 | 16.37 | 256.5 | -7.72 | 320 | 16.28 | 250.1 | -7.70 |
| 330 |  |  |  | 330 | 16.30 | 249.6 | -7.67 |
| 340 |  |  |  | 340 | 16.32 | 249.1 | -7.64 |
| 350 |  |  |  | 350 | 16.33 | 248.7 | -7.63 |
| 360 |  |  |  | 360 | 16.34 | 248.3 | -7.61 |
| 370 |  |  |  | 370 | 16.34 | 247.9 | -7.60 |
| 380 |  |  |  | 380 | 16.35 | 247.6 | -7.59 |
| 390 |  |  |  | 390 | 16.34 | 247.3 | -7.59 |
| 400 |  |  |  | 400 | 16.34 | 247.1 | -7.59 |

TWO ANTENNAS VERTICALLY STACKED AT 290CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2x QY20905XL3D4 | No losss | 16.31 | 19.95 | 28.70 | 9.63 | 42.6 | 19.0 | 248.0 | -7.63 | 139.800- 148.100 |
|  | Aluminum | 16.26 | 19.95 | 28.64 | 9.65 | 42.6 | 19.0 | 248.5 | -7.69 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 290+290+290CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL3D4 | No losss | 19.33 | 20.60 | 24.30 | 12.43 | 43.0 | 8.6 | 231.8 | -4.32 | 139.800- 148.100 |
|  | Aluminum | 19.27 | 20.59 | 24.32 | 12.42 | 43.0 | 8.6 | 232.5 | -4.39 |  |

FOUR ANTENNAS IN H- STACK, H=374 CM, V=280 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL3D4 | No losss | 19.32 | 19.67 | 6.62 | 10.31 | 14.6 | 19.6 | 247.3 | -4.61 | 139.800- 148.100 |
|  | Aluminum | 19.27 | 19.67 | 6.63 | 10.33 | 14.6 | 19.6 | 247.9 | -4.67 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE2 from 900 to 886 mm

- Change length of D1 from 940 to 936 mm

- Change position of D1 for +10 mm.

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 13.33 | 18.85 | 25.89 | 16.99 | 42.0 | 46.2 | 246.0 | -10.58 | 139.600 – 146.800 |
|  |  | Aluminum | 13.28 | 18.84 | 25.87 | 16.98 | 42.0 | 46.0 | 246.7 | -10.64 |  |
|  | Vertical part | No losss | 13.33 | 16.83 | 15.76 | 25.54 | 45.8 | 41.8 | 243.9 | -10.54 | 140.700 – 149.000 |
|  |  | Aluminum | 13.28 | 16.82 | 15.76 | 25.43 | 45.8 | 41.6 | 244.7 | -10.61 |  |