ANTENNA QY20905XL4D3

Good stacking characteristics at moderate vertical distances and big horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
|  | QY20905XL4D3 | 9 | 4205 | 14.23 | 24.07 | 18.99 | 18.05 | 37.2 | 40.4 | 236.4 | -9.51 | 4300 |

* ΔF given for SWR=1,5

|  | PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- | --- |
| Element material |  | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods |  | 14.15 | 244.9 | -10.45 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20905XL4D3 | 4205 | No loss | 14.23 | 236.4 | -9.51 | 142.600- 146.600 |
|  |  | Aluminum | 14.15 | 237.8 | -9.61 |  |
| EF0208-5 | 3900 | No loss | 13.53 | 248.2 | -10.42 | 141.220-145.490 |
|  |  | Aluminum | 13.45 | 249.2 | -10.52 |  |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | DQ2 | DQ3 | DQ4 | REMARKS: - All elements made of Al rods Ǿ5 mm - Height of all rectangles 520 mm - All dimensions given in milimetres |
| Pos | 0 | 265 | 339 | 425 | 732 | 1265 | 1955 | 2719 | 3511 | 4205 |  |
| Length | 1016 | 1016 | 840 | 944 | 954 | 930 | 906 | 512 | 502 | 490 |  |

Building hints: No boom correction included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20905XL4D3 - STACKING TABLE (including loss for aluminum – AGT = -0.09 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 15.15 | 238.0 | -8.62 | 100 |  |  |  |
| 110 | 15.33 | 230.3 | -8.29 | 110 | 14.52 | 262.9 | -9.68 |
| 120 | 15.48 | 226.3 | -8.07 | 120 | 14.99 | 257.8 | -9.12 |
| 130 | 15.61 | 225.4 | -7.92 | 130 | 15.26 | 251.6 | -8.75 |
| 140 | 15.72 | 225.7 | -7.82 | 140 | 15.47 | 246.5 | -8.45 |
| 150 | 15.83 | 225.4 | -7.70 | 150 | 15.63 | 242.6 | -8.22 |
| 160 | 15.94 | 224.6 | -7.57 | 160 | 15.77 | 239.8 | -8.03 |
| 170 | 16.04 | 224.9 | -7.48 | 170 | 15.89 | 237.8 | -7.87 |
| 180 | 16.13 | 226.5 | -7.42 | 180 | 16.00 | 236.6 | -7.74 |
| 190 | 16.22 | 228.6 | -7.37 | 190 | 16.10 | 235.8 | -7.63 |
| 200 | 16.30 | 230.6 | -7.33 | 200 | 16.20 | 235.4 | -7.52 |
| 210 | 16.39 | 231.8 | -7.26 | 210 | 16.28 | 235.4 | -7.44 |
| 220 | 16.47 | 232.1 | -7.19 | 220 | 16.37 | 235.4 | -7.35 |
| 230 | 16.57 | 231.7 | -7.08 | 230 | 16.45 | 235.7 | -7.27 |
| 240 | 16.66 | 231.1 | -6.98 | 240 | 16.53 | 235.9 | -7.20 |
| 250 | 16.75 | 230.6 | -6.88 | 250 | 16.61 | 236.2 | -7.12 |
| 260 | 16.83 | 230.5 | -6.80 | 260 | 16.69 | 236.5 | -7.05 |
| 270 | 16.91 | 231.0 | -6.73 | 270 | 16.77 | 236.8 | -6.97 |
| 280 | 16.98 | 232.0 | -6.68 | 280 | 16.84 | 237.0 | -6.91 |
| 290 | 17.04 | 233.5 | -6.64 | 290 | 16.90 | 237.2 | -6.85 |
| 300 | 17.09 | 235.2 | -6.63 | 300 | 16.96 | 237.4 | -6.79 |
| 310 | 17.14 | 237.1 | -6.61 | 310 | 17.01 | 237.6 | -6.75 |
| 320 | 17.17 | 239.1 | -6.62 | 320 | 17.05 | 237.7 | -6.71 |
| 330 | 17.20 | 241.1 | -6.62 | 330 | 17.09 | 237.9 | -6.67 |
| 340 | 17.22 | 243.0 | -6.64 | 340 | 17.12 | 238.0 | -6.65 |
| 350 | 17.23 | 244.8 | -6.66 | 350 | 17.15 | 238.1 | -6.62 |
| 360 | 17.24 | 246.4 | -6.68 | 360 | 17.17 | 238.2 | -6.60 |
| 370 |  |  |  | 370 | 17.18 | 238.3 | -6.59 |
| 380 |  |  |  | 380 | 17.20 | 238.4 | -6.57 |
| 390 |  |  |  | 390 | 17.20 | 238.5 | -6.57 |
| 400 |  |  |  | 400 | 17.21 | 238.6 | -6.57 |

TWO ANTENNAS VERTICALLY STACKED AT 310CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2x QY20905XL4D3 | No losss | 17.23 | 25.07 | 20.28 | 10.68 | 37.6 | 17.6 | 235.9 | -6.50 | 142.600- 146.600 |
|  | Aluminum | 17.14 | 24.85 | 20.23 | 10.72 | 37.4 | 17.6 | 237.1 | -6.61 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 310+310+310CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL4D3 | No losss | 20.23 | 25.83 | 20.98 | 13.11 | 37.8 | 8.0 | 224.4 | -3.28 | 142.600- 146.600 |
|  | Aluminum | 20.14 | 25.58 | 20.93 | 13.12 | 37.8 | 8.0 | 226.0 | -3.40 |  |

FOUR ANTENNAS IN H- STACK, H=370 CM, V=304 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL4D3 | No losss | 20.26 | 25.29 | 8.45 | 10.99 | 14.6 | 18.0 | 238.3 | -3.51 | 142.600- 146.600 |
|  | Aluminum | 20.17 | 25.07 | 8.47 | 11.02 | 14.6 | 18.0 | 239.5 | -4.62 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of D1 from 944 to 948 mm

- Change length of D2 from 954 to 956 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 14.23 | 24.07 | 18.99 | 18.05 | 37.2 | 40.4 | 236.4 | -9.51 | 142.300 – 146.600 |
|  |  | Aluminum | 14.15 | 23.87 | 28.96 | 18.06 | 37.2 | 40.4 | 237.8 | -9.61 |  |
|  | Vertical part | No losss | 14.31 | 19.62 | 16.77 | 18.38 | 39.4 | 36.6 | 238.6 | -9.47 | 143.100 – 146.000 |
|  |  | Aluminum | 14.22 | 19.51 | 16.77 | 18.36 | 39.4 | 36.6 | 240.1 | -9.58 |  |