ANTENNA QY20905XL5D4

Good stacking characteristics at moderate vertical distances and big horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20905XL5D4 | 9 | 3205 | 13.16 | 32.29 | 20.37 | 17.61 | 42.2 | 46.4 | 248.2 | -10.79 | 8400 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 13.10 | 249.0 | -10.86 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20905XL5D4 | 3205 | No loss | 13.16 | 248.2 | -10.79 | 139.900- 146.500 |
|  |  | Aluminum | 13.10 | 249.0 | -10.86 |  |
| EF0207-5 | 3274 | No loss | 12.91 | 283.0 | -12.61 | 140.400-145.100 |
|  |  | Aluminum | 12.85 | 256.9 | -11.25 |  |
| G0KSC 7ELE EU | 2894 | No loss | 12.89 | 260.5 | -11.27 | 141.300 - 145.400 |
|  |  | Aluminum | 12.83 | 260.9 | -11.34 |  |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | DQ1 | DQ2 | DQ3 | DQ4 | REMARKS: - All elements made of Al rods Ǿ5 mm - Height of all rectangles 500 mm - All dimensions given in milimetres |
| Pos | 0 | 221 | 330 | 415 | 687 | 1142 | 1655 | 2075 | 2625 | 3205 |  |
| Length | 1024 | 1024 | 870 | 946 | 946 | 922 | 538 | 504 | 510 | 490 |  |

Building hints: No boom correction included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

| QY20905XL5D4 (including loss for aluminum – AGT = -0.07 to -.06 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 14.32 | 231.1 | -9.32 | 100 |  |  |  |
| 110 | 14.49 | 227.5 | -9.08 | 110 | 14.00 | 261.3 | -10.17 |
| 120 | 14.63 | 226.1 | -8.91 | 120 | 14.28 | 253.3 | -9.76 |
| 130 | 14.76 | 226.5 | -8.79 | 130 | 14.49 | 248.7 | -9.47 |
| 140 | 14.88 | 227.3 | -8.69 | 140 | 14.66 | 245.8 | -9.25 |
| 150 | 14.99 | 227.1 | -8.57 | 150 | 14.81 | 244.2 | -9.07 |
| 160 | 15.11 | 226.4 | -8.44 | 160 | 14.94 | 243.6 | -8.93 |
| 170 | 15.22 | 226.6 | -8.33 | 170 | 15.06 | 243.7 | -8.81 |
| 180 | 15.31 | 227.7 | -8.26 | 180 | 15.17 | 244.3 | -8.71 |
| 190 | 15.41 | 228.6 | -8.18 | 190 | 15.28 | 245.2 | -8.61 |
| 200 | 15.50 | 228.9 | -8.10 | 200 | 15.38 | 246.3 | -8.53 |
| 210 | 15.60 | 228.7 | -7.99 | 210 | 15.47 | 247.4 | -8.46 |
| 220 | 15.70 | 228.7 | -7.89 | 220 | 15.56 | 248.4 | -8.39 |
| 230 | 15.80 | 229.3 | -7.80 | 230 | 15.65 | 249.3 | -8.32 |
| 240 | 15.88 | 230.9 | -7.75 | 240 | 15.73 | 250.1 | -8.25 |
| 250 | 15.96 | 233.5 | -7.72 | 250 | 15.81 | 250.7 | -8.18 |
| 260 | 16.02 | 236.9 | -7.73 | 260 | 15.88 | 251.1 | -8.12 |
| 270 | 16.07 | 241.0 | -7.75 | 270 | 15.93 | 251.4 | -8.07 |
| 280 | 16.12 | 245.3 | -7.78 | 280 | 15.99 | 251.4 | -8.01 |
| 290 | 16.15 | 249.7 | -7.83 | 290 | 16.03 | 251.4 | -7.97 |
| 300 | 16.17 | 254.0 | -7.88 | 300 | 16.07 | 251.3 | -7.93 |
| 310 | 16.19 | 257.9 | -7.92 | 310 | 16.10 | 251.0 | -7.90 |
| 320 | 16.20 | 261.4 | -7.97 | 320 | 16.12 | 250.7 | -7.87 |
| 330 |  |  |  | 330 | 16.14 | 250.4 | -7.85 |
| 340 |  |  |  | 340 | 16.15 | 250.1 | -7.83 |
| 350 |  |  |  | 350 | 16.16 | 249.8 | -7.82 |
|  |  |  |  | 360 | 16.16 | 249.6 | -7.81 |

TWO ANTENNAS VERTICALLY STACKED AT 250CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY20905XL5D4 | No losss | 16.02 | 33.32 | 23.40 | 13.05 | 42.2 | 21.4 | 232.5 | -7.64 | 139.900- 146.500 |
|  | Aluminum | 15.96 | 33.03 | 23.35 | 13.08 | 42.2 | 21.4 | 233.5 | -7.72 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 250+250+250CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL5D4 | No losss | 18.93 | 32.66 | 25.19 | 12.75 | 42.3 | 10.0 | 224.1 | -4.57 | 139.900- 146.500 |
|  | Aluminum | 18.87 | 32.46 | 25.13 | 12.77 | 42.2 | 10.0 | 225.3 | -4.66 |  |

FOUR ANTENNAS IN H- STACK, H=340 CM, V=242 CM, LOSS INCLUDED FOR ALUMINUM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL5D4 | No losss | 19.05 | 33.43 | 8.07 | 13.86 | 16.0 | 22.0 | 230.7 | -4.58 | 139.900- 146.500 |
|  | Aluminum | 18.99 | 33.14 | 8.09 | 13.89 | 16.0 | 22.0 | 231.7 | -4.66 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- Vertical director D1 should be shortened for 4 mm

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE1 from 870 to 916 mm

- Change length of D2 from 946 to 948 mm

- Change position of D1 for -8 mm.

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 13.16 | 33.29 | 20.37 | 17.61 | 42.2 | 46.4 | 248.2 | -10.79 | 139.900- 146.500 |
|  |  | Aluminum | 13.10 | 32.83 | 20.32 | 17.60 | 42.2 | 46.2 | 249.0 | -10.86 |  |
|  | Vertical part | No losss | 13.14 | 36.21 | 17.39 | 21.70 | 46.6 | 42.4 | 246.3 | -10.77 | 138.600 – 148.900 |
|  |  | Aluminum | 13.08 | 36.39 | 17.39 | 21.65 | 46.4 | 42.2 | 247.2 | -10.85 |  |