ANTENNA QY20905XL9D4

Good stacking characteristics at small vertical and big horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20905XL9D4 | 9 | 3805 | 13.47 | 23.43 | 24.65 | 17.69 | 41.0 | 44.6 | 239.0 | -10.31 | 6750 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 13.42 | 239.8 | -10.38 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20905XL9D4 | 3805 | No loss | 13.47 | 239.0 | -10.31 | 139.900- 146.550 |
|  |  | Aluminum | 13.42 | 239.8 | -10.38 |  |
| EF0208-5 | 3900 | No loss | 13.53 | 248.2 | -10.42 | 141.220-145.490 |
|  |  | Aluminum | 13.45 | 249.2 | -10.52 |  |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | DQ1 | DQ2 | DQ3 | DQ4 | REMARKS: - All elements made of Al rods Ǿ5 mm - Height of all rectangles 450 mm - All dimensions given in milimetres |
| Pos | 0 | 180 | 335 | 420 | 706 | 1065 | 1585 | 2257 | 3075 | 3805 |  |
| Length | 1020 | 1020 | 820 | 945 | 934 | 912 | 578 | 550 | 524 | 502 |  |

Building hints: No boom correction included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20905XL9D4 – STACKING TABLE (including loss for aluminum – AGT = -0.06 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 14.78 | 223.1 | -8.70 | 100 |  |  |  |
| 110 | 14.92 | 219.8 | -8.50 | 110 |  |  |  |
| 120 | 15.04 | 218.8 | -8.36 | 120 | 14.76 | 239.7 | -9.04 |
| 130 | 15.14 | 220.2 | -8.29 | 130 | 14.91 | 236.5 | -8.83 |
| 140 | 15.22 | 222.8 | -8.26 | 140 | 15.04 | 234.8 | -8.67 |
| 150 | 15.30 | 224.6 | -8.21 | 150 | 15.15 | 234.2 | -8.55 |
| 160 | 15.38 | 224.8 | -8.14 | 160 | 15.25 | 234.3 | -8.45 |
| 170 | 15.47 | 224.8 | -8.05 | 170 | 15.35 | 234.9 | -8.36 |
| 180 | 15.56 | 225.4 | -7.97 | 180 | 15.45 | 235.9 | -8.28 |
| 190 | 15.64 | 226.1 | -7.90 | 190 | 15.54 | 237.1 | -8.21 |
| 200 | 15.73 | 226.6 | -7.82 | 200 | 15.63 | 238.3 | -8.14 |
| 210 | 15.81 | 226.6 | -7.74 | 210 | 15.72 | 239.6 | -8.07 |
| 220 | 15.91 | 226.4 | -7.64 | 220 | 15.81 | 240.6 | -8.00 |
| 230 | 16.00 | 226.5 | -7.55 | 230 | 15.89 | 241.6 | -7.94 |
| 240 | 16.08 | 227.3 | -7.49 | 240 | 15.98 | 242.3 | -7.86 |
| 250 | 16.17 | 228.7 | -7.42 | 250 | 16.05 | 242.9 | -7.80 |
| 260 | 16.24 | 230.9 | -7.40 | 260 | 16.13 | 243.2 | -7.73 |
| 270 | 16.31 | 233.8 | -7.38 | 270 | 16.19 | 243.4 | -7.67 |
| 280 | 16.36 | 237.0 | -7.39 | 280 | 16.25 | 243.4 | -7.61 |
| 290 | 16.41 | 240.4 | -7.40 | 290 | 16.30 | 243.3 | -7.56 |
| 300 | 16.44 | 243.9 | -7.43 | 300 | 16.35 | 243.1 | -7.51 |
| 310 |  |  |  | 310 | 16.38 | 242.8 | -7.47 |
| 320 |  |  |  | 320 | 16.41 | 242.5 | -7.44 |
| 330 |  |  |  | 330 | 16.44 | 242.2 | -7.40 |
| 340 |  |  |  | 340 | 16.45 | 241.8 | -7.39 |
| 350 |  |  |  | 350 | 16.47 | 241.5 | -7.36 |
| 360 |  |  |  | 360 | 16.47 | 241.2 | -7.35 |
| 370 |  |  |  | 370 | 16.48 | 241.0 | -7.34 |
| 380 |  |  |  | 380 |  |  |  |
| 390 |  |  |  | 390 |  |  |  |
| 400 |  |  |  | 400 |  |  |  |

TWO ANTENNAS VERTICALLY STACKED AT 270CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2x QY20905XL9D4 | No losss | 16.36 | 24.84 | 28.65 | 11.58 | 41.8 | 20.0 | 233.0 | -7.31 | 139.900- 146.550 |
|  | Aluminum | 16.31 | 24.84 | 28.57 | 11.61 | 41.6 | 20.0 | 233.8 | -7.38 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 270+270+270CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL9D4 | No losss | 19.30 | 25.75 | 29.56 | 12.86 | 42.0 | 9.2 | 220.6 | -4.14 | 139.900- 146.550 |
|  | Aluminum | 19.25 | 25.75 | 29.50 | 12.87 | 42.0 | 9.2 | 221.7 | -4.21 |  |

FOUR ANTENNAS IN H- STACK, H=370 CM, V=260 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20905XL9D4 | No losss | 19.37 | 24.36 | 7.02 | 12.58 | 14.8 | 20.8 | 231.5 | -4.28 | 139.900- 146.550 |
|  | Aluminum | 19.32 | 24.36 | 7.04 | 12.61 | 14.6 | 20.6 | 232.4 | -4.34 |  |

XPOL VERSION

I do not recommend this antenna for XPOL, though it can be interesting to someone for its very great bandwidth

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE1 from 1020 to 1000 mm

- Change length of DE2 from 820 to 860 mm

- Change length of D1 from 945 to 934 mm

- Change length of D2 from 934 to 932 mm

- Change position of D1 for +15 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 13.47 | 23.43 | 24.65 | 17.69 | 41.0 | 44.6 | 239.0 | -10.31 | 142.300 – 146.600 |
|  |  | Aluminum | 13.42 | 21.39 | 22.18 | 16.91 | 41.0 | 44.6 | 239.8 | -10.38 |  |
|  | Vertical part | No losss | 12.80 | 18.57 | 16.36 | 29.33 | 49.4 | 44.0 | 245.3 | -11.10 | 140.700 – 149.800 |
|  |  | Aluminum | 12.75 | 18.54 | 16.36 | 29.30 | 49.4 | 44.0 | 247.4 | -11.22 |  |