ANTENNA QY20906XL3D3

Good stacking characteristics at moderate vertical distances and big horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20906XL3D3 | 9 | 4255 | 14.18 | 21.08 | 19.17 | 18.92 | 37.4 | 40.8 | 234.7 | -9.53 | 3700 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 14.10 | 236.0 | -9.63 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | DQ1 | DQ2 | DQ3 | REMARKS: -All elements made of Al rods Ǿ6 mm - The height of the rectangle is 500 mm - All dimensions given in milimetres |
| Pos | 0 | 210 | 350 | 426 | 718 | 1265 | 1965 | 2705 | 3485 | 4255 |  |
| Length | 1016 | 1000 | 896 | 945 | 953 | 928 | 906 | 530 | 510 | 490 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20906XL3D3 | 4255 | No loss | 14.18 | 234.7 | -9.53 | 142.450- 146.150 |
|  |  | Aluminum | 14.10 | 236.0 | -9.63 |  |
| G0KSC 9ELWB | 4447 | No loss | 14.12 | 239.6 | -9.68 | 142.700- 145.550 |
|  |  | Aluminum | 14.04 | 240.9 | -9.78 |  |
| EF0209-5 | 4490 | No loss | 14.06 | 239.8 | -9.74 | 140.100 - 145.480 |
|  |  | Aluminum | 13.99 | 240.9 | -9.83 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20906XL3D3 - STACKING TABLE (including loss for aluminum – AGT = -0.09 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 15.10 | 234.6 | -8.60 | 100 |  |  |  |
| 110 | 15.27 | 229.0 | -8.33 | 110 |  |  |  |
| 120 | 15.42 | 226.2 | -8.13 | 120 | 14.84 | 254.6 | -9.22 |
| 130 | 15.55 | 225.4 | -7.98 | 130 | 15.17 | 248.7 | -8.79 |
| 140 | 15.67 | 224.9 | -7.85 | 140 | 15.39 | 244.0 | -8.48 |
| 150 | 15.78 | 223.7 | -7.72 | 150 | 15.57 | 240.4 | -8.24 |
| 160 | 15.90 | 222.3 | -7.57 | 160 | 15.71 | 237.7 | -8.05 |
| 170 | 16.01 | 221.8 | -7.45 | 170 | 15.84 | 235.8 | -7.89 |
| 180 | 16.12 | 222.2 | -7.35 | 180 | 15.96 | 234.5 | -7.74 |
| 190 | 16.22 | 223.0 | -7.26 | 190 | 16.07 | 233.7 | -7.62 |
| 200 | 16.31 | 223.9 | -7.19 | 200 | 16.17 | 233.1 | -7.51 |
| 210 | 16.40 | 224.5 | -7.11 | 210 | 16.26 | 232.9 | -7.41 |
| 220 | 16.50 | 224.7 | -7.02 | 220 | 16.35 | 232.9 | -7.32 |
| 230 | 16.59 | 224.8 | -6.93 | 230 | 16.44 | 233.1 | -7.24 |
| 240 | 16.68 | 225.1 | -6.84 | 240 | 16.52 | 233.5 | -7.16 |
| 250 | 16.76 | 225.7 | -6.77 | 250 | 16.60 | 233.8 | -7.09 |
| 260 | 16.83 | 226.9 | -6.73 | 260 | 16.68 | 234.3 | -7.02 |
| 270 | 16.90 | 228.6 | -6.69 | 270 | 16.75 | 234.7 | -6.96 |
| 280 | 16.96 | 230.7 | -6.67 | 280 | 16.82 | 235.1 | -6.89 |
| 290 | 17.01 | 233.2 | -6.67 | 290 | 16.88 | 235.5 | -6.84 |
| 300 | 17.06 | 235.8 | -6.66 | 300 | 16.93 | 235.8 | -6.80 |
| 310 | 17.09 | 238.4 | -6.68 | 310 | 16.98 | 236.2 | -6.75 |
| 320 | 17.12 | 240.9 | -6.70 | 320 | 17.01 | 236.4 | -6.73 |
| 330 | 17.14 | 243.2 | -6.72 | 330 | 17.05 | 236.6 | -6.69 |
| 340 |  |  |  | 340 | 17.08 | 236.8 | -6.66 |
| 350 |  |  |  | 350 | 17.10 | 237.0 | -6.65 |
| 360 |  |  |  | 360 | 17.12 | 237.1 | -6.63 |
| 370 |  |  |  | 370 | 17.13 | 237.2 | -6.62 |
| 380 |  |  |  | 380 | 17.14 | 237.3 | -6.61 |
| 390 |  |  |  | 390 | 17.15 | 237.4 | -6.60 |
| 400 |  |  |  | 400 | 17.15 | 237.4 | -6.61 |

TWO ANTENNAS VERTICALLY STACKED AT 280CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2x QY20906XL3D3 | No losss | 17.04 | 22.65 | 21.01 | 13.17 | 37.4 | 19.2 | 229.4 | -6.57 | 142.450- 146.150 |
|  | Aluminum | 16.96 | 22.57 | 20.96 | 13.20 | 37.4 | 19.2 | 230.7 | -6.67 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 280+280+280CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20906XL3D3 | No losss | 19.97 | 23.75 | 22.29 | 13.06 | 37.4 | 8.8 | 219.7 | -3.45 | 142.450- 146.150 |
|  | Aluminum | 19.89 | 23.66 | 22.24 | 13.07 | 37.4 | 8.8 | 221.2 | -3.56 |  |

FOUR ANTENNAS IN H- STACK, H=390 CM, V=280 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20906XL3D3 | No losss | 20.11 | 22.46 | 7.74 | 13.06 | 14.0 | 19.2 | 232.4 | -3.55 | 142.450- 146.150 |
|  | Aluminum | 20.03 | 22.38 | 7.77 | 13.10 | 13.9 | 19.2 | 233.6 | -3.65 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of D3 from 928 to 923 mm

- Change length of DE2 from 896 to 890 mm

- Change position of D1 for +1 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 14.18 | 21.08 | 19.17 | 18.92 | 37.4 | 40.8 | 234.7 | -9.53 | 142.450- 146.150 |
|  |  | Aluminum | 14.10 | 21.01 | 19.12 | 18.91 | 37.4 | 40.8 | 236.0 | -9.63 |  |
|  | Vertical part | No losss | 14.15 | 23.06 | 19.46 | 20.34 | 41.2 | 37.8 | 234.0 | -9.54 | 142.200 – 146.600 |
|  |  | Aluminum | 14.07 | 22.96 | 19.45 | 20.30 | 41.2 | 37.6 | 235.3 | -9.65 |  |