ANTENNA QY20908XL4D2

Good stacking characteristics at moderate vertical and big horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY20908XL4D2 | 9 | 4265 | 14.15 | 20.35 | 20.67 | 18.07 | 37.4 | 41.4 | 236.1 | -9.58 | 3500 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 14.08 | 237.0 | -9.67 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | REMARKS: : -All elements made of Al rods Ǿ8 mm - The height of the rectangle is 500 mm - All dimensions given in milimetres |
| Pos | 0 | 182 | 330 | 407 | 690 | 1255 | 1943 | 2695 | 3525 | 4625 |  |
| Length | 1010 | 1010 | 890 | 940 | 948 | 918 | 902 | 884 | 510 | 490 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY20908XL4D2 | 4265 | No loss | 14.15 | 236.1 | -9.58 | 142.700- 146.200 |
|  |  | Aluminum | 14.08 | 237.0 | -9.67 |  |
| GOKSC 9EL WB | 4447 | No loss | 14.12 | 239.6 | -9.68 | 142.700- 145.550 |
|  |  | Aluminum | 14.04 | 240.9 | -9.78 |  |
| EF0208-5 | 4490 | No loss | 14.06 | 239.8 | -9.74 | 140.100- 145.480 |
|  |  | Aluminum | 13.99 | 240.9 | -9.83 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20908XL4D2 - STACKING TABLE (including loss for aluminum – AGT = -0.07 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 15.19 | 232.5 | -8.48 | 100 |  |  |  |
| 110 | 15.34 | 227.4 | -8.23 | 110 |  |  |  |
| 120 | 15.48 | 224.2 | -8.03 | 120 | 14.84 | 261.0 | -9.33 |
| 130 | 15.61 | 222.5 | -7.86 | 130 | 15.17 | 252.5 | -8.85 |
| 140 | 15.72 | 221.3 | -7.73 | 140 | 15.39 | 246.4 | -8.53 |
| 150 | 15.83 | 220.3 | -7.60 | 150 | 15.56 | 242.1 | -8.28 |
| 160 | 15.93 | 220.0 | -7.49 | 160 | 15.71 | 239.0 | -8.07 |
| 170 | 16.02 | 220.9 | -7.42 | 170 | 15.83 | 236.8 | -7.91 |
| 180 | 16.12 | 222.2 | -7.35 | 180 | 15.95 | 235.3 | -7.77 |
| 190 | 16.21 | 223.6 | -7.28 | 190 | 16.05 | 234.3 | -7.65 |
| 200 | 16.30 | 224.6 | -7.21 | 200 | 16.15 | 233.7 | -7.54 |
| 210 | 16.40 | 225.0 | -7.12 | 210 | 16.25 | 233.4 | -7.43 |
| 220 | 16.49 | 225.0 | -7.03 | 220 | 16.34 | 233.4 | -7.34 |
| 230 | 16.59 | 225.0 | -6.93 | 230 | 16.43 | 233.6 | -7.25 |
| 240 | 16.68 | 225.2 | -6.85 | 240 | 16.51 | 234.0 | -7.18 |
| 250 | 16.76 | 226.0 | -6.78 | 250 | 16.59 | 234.4 | -7.11 |
| 260 | 16.84 | 227.4 | -6.73 | 260 | 16.67 | 234.9 | -7.04 |
| 270 | 16.90 | 229.4 | -6.71 | 270 | 16.74 | 235.4 | -6.98 |
| 280 | 16.96 | 231.9 | -6.69 | 280 | 16.80 | 236.0 | -6.93 |
| 290 | 17.01 | 234.6 | -6.69 | 290 | 16.86 | 236.4 | -6.88 |
| 300 | 17.06 | 237.4 | -6.70 | 300 | 16.91 | 236.9 | -6.83 |
|  |  |  |  | 310 | 16.96 | 237.2 | -6.79 |
|  |  |  |  | 320 | 17.00 | 237.6 | -6.76 |
|  |  |  |  | 330 | 17.03 | 237.8 | -6.73 |
|  |  |  |  | 340 | 17.06 | 238.1 | -6.71 |
|  |  |  |  | 350 | 17.08 | 238.3 | -6.69 |
|  |  |  |  | 360 | 17.10 | 238.5 | -6.68 |
|  |  |  |  | 370 | 17.11 | 238.7 | -6.67 |
|  |  |  |  | 380 | 17.12 | 238.8 | -6.66 |
|  |  |  |  | 390 | 17.13 | 238.9 | -6.65 |
|  |  |  |  | 400 | 17.13 | 239.0 | -6.65 |

TWO ANTENNAS VERTICALLY STACKED AT 280CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2x QY20908XL4D2 | No losss | 17.03 | 21.32 | 23.03 | 12.85 | 37.4 | 19.2 | 230.8 | -6.60 | 142.700- 146.200 |
|  | Aluminum | 16.96 | 21.25 | 23.02 | 12.87 | 37.4 | 19.2 | 231.9 | -6.69 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 280+280+280CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20908XL4D2 | No losss | 19.96 | 22.03 | 24.25 | 13.01 | 37.6 | 8.8 | 219.9 | -3.46 | 142.700- 146.200 |
|  | Aluminum | 19.90 | 21.96 | 24.20 | 13.01 | 37.6 | 8.8 | 221.0 | -3.54 |  |

FOUR ANTENNAS IN H- STACK, H=380 CM, V=270 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY20908XL4D2 | No losss | 20.03 | 21.16 | 8.12 | 13.64 | 14.2 | 19.8 | 231.8 | -3.62 | 142.700- 146.200 |
|  | Aluminum | 19.97 | 21.10 | 8.14 | 13.67 | 14.2 | 19.6 | 232.8 | -3.70 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm back regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of D1 from 940 to 938 mm

In this case the SWR diagram will be like this:

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 14.15 | 20.35 | 20.67 | 18.07 | 37.4 | 41.4 | 236.1 | -9.58 | 142.700- 146.200 |
|  |  | Aluminum | 14.08 | 20.29 | 20.65 | 18.04 | 37.4 | 41.4 | 237.0 | -9.67 |  |
|  | Vertical part | No losss | 14.16 | 19.67 | 18.50 | 20.95 | 41.4 | 37.4 | 230.7 | -9.47 | 142.600 – 146.000 |
|  |  | Aluminum | 14.10 | 19.62 | 18.47 | 20.93 | 41.2 | 37.4 | 231.7 | -9.55 |  |