ANTENNA QY21008XL4D3

Good stacking characteristics at any vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21008XL4D3 | 10 | 5105 | 14.81 | 23.03 | 18.43 | 17.98 | 34.6 | 37.4 | 230.8 | -8.82 | 3380 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 14.74 | 232.1 | -8.92 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | REMARKS: : -All elements made of Al rods Ǿ8 mm - The height of the rectangle is 500 mm - All dimensions given in milimetres |
| Pos | 0 | 190 | 330 | 406 | 705 | 1255 | 1943 | 2705 | 3535 | 4315 | 5105 |  |
| Length | 1012 | 1000 | 920 | 942 | 948 | 924 | 904 | 884 | 526 | 502 | 484 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21008XL4D3 | 5105 | No loss | 14.81 | 230.8 | -8.82 | 142.700- 146.080 |
|  |  | Aluminum | 14.74 | 232.1 | -8.92 |  |
| EF0210-5 | 5290 | No loss | 14.77 | 233.8 | -8.92 | 140.190- 145.450 |
|  |  | Aluminum | 14.67 | 235.4 | -9.05 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY20908XL4D3 - STACKING TABLE (including loss for aluminum – AGT = -0.08 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 15.52 | 236.3 | -8.21 | 100 |  |  |  |
| 110 | 15.71 | 230.2 | -7.91 | 110 |  |  |  |
| 120 | 15.88 | 226.9 | -7.68 | 120 |  |  |  |
| 130 | 16.02 | 225.6 | -7.51 | 130 |  |  |  |
| 140 | 16.15 | 224.6 | -7.36 | 140 | 15.82 | 245.7 | -8.08 |
| 150 | 16.27 | 222.7 | -7.21 | 150 | 16.03 | 241.7 | -7.80 |
| 160 | 16.40 | 220.4 | -7.03 | 160 | 16.19 | 238.7 | -7.59 |
| 170 | 16.51 | 219.1 | -6.90 | 170 | 16.33 | 236.4 | -7.41 |
| 180 | 16.62 | 219.1 | -6.79 | 180 | 16.45 | 234.6 | -7.25 |
| 190 | 16.72 | 220.1 | -6.71 | 190 | 16.56 | 233.2 | -7.12 |
| 200 | 16.81 | 221.7 | -6.65 | 200 | 16.67 | 232.1 | -6.99 |
| 210 | 16.90 | 223.4 | -6.59 | 210 | 16.76 | 231.2 | -6.88 |
| 220 | 16.98 | 224.7 | -6.54 | 220 | 16.85 | 230.5 | -6.78 |
| 230 | 17.07 | 225.5 | -6.46 | 230 | 16.94 | 230.0 | -6.68 |
| 240 | 17.16 | 225.8 | -6.38 | 240 | 17.02 | 229.6 | -6.59 |
| 250 | 17.24 | 225.9 | -6.30 | 250 | 17.10 | 229.3 | -6.50 |
| 260 | 17.33 | 226.1 | -6.21 | 260 | 17.18 | 229.1 | -6.42 |
| 270 | 17.40 | 226.6 | -6.15 | 270 | 17.26 | 229.0 | -6.34 |
| 280 | 17.47 | 227.3 | -6.10 | 280 | 17.33 | 229.0 | -6.27 |
| 290 | 17.54 | 228.3 | -6.05 | 290 | 17.40 | 229.1 | -6.20 |
| 300 | 17.59 | 229.6 | -6.02 | 300 | 17.46 | 229.2 | -6.14 |
| 310 | 17.64 | 231.2 | -6.00 | 310 | 17.52 | 229.4 | -6.09 |
| 320 | 17.68 | 232.8 | -5.99 | 320 | 17.57 | 229.6 | -6.04 |
| 330 | 17.72 | 234.5 | -5.98 | 330 | 17.61 | 229.9 | -6.00 |
| 340 | 17.74 | 236.1 | -5.99 | 340 | 17.65 | 230.2 | -5.97 |
| 350 |  |  |  | 350 | 17.68 | 230.5 | -5.95 |
| 360 |  |  |  | 360 | 17.71 | 230.9 | -5.92 |
| 370 |  |  |  | 370 | 17.73 | 231.3 | -5.91 |
| 380 |  |  |  | 380 | 17.75 | 231.7 | -5.90 |
| 390 |  |  |  | 390 | 17.76 | 232.1 | -5.90 |
| 400 |  |  |  | 400 | 17.77 | 232.5 | -5.89 |

TWO ANTENNAS VERTICALLY STACKED AT 330CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21108XL4D3 | No losss | 17.79 | 19.61 | 18.90 | 10.92 | 34.8 | 16.2 | 233.3 | -5.89 | 142.700- 146.080 |
|  | Aluminum | 17.72 | 19.62 | 18.87 | 10.95 | 34.8 | 16.2 | 234.5 | -5.98 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 330+330+330CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21108XL4D4 | No losss | 20.79 | 23.88 | 20.29 | 13.09 | 35.0 | 7.4 | 227.9 | -2.79 | 142.700- 146.080 |
|  | Aluminum | 20.71 | 23.80 | 20.25 | 13.09 | 35.0 | 7.4 | 229.2 | -2.89 |  |

FOUR ANTENNAS IN H- STACK, H=380 CM, V=330 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21108XL4D4 | No losss | 20.82 | 23.29 | 9.24 | 10.81 | 14.2 | 16.4 | 235.5 | -2.90 | 142.700- 146.080 |
|  | Aluminum | 20.75 | 23.23 | 9.26 | 10.84 | 14.0 | 16.2 | 236.6 | -2.99 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of D1 from 942 to 944 mm

- Change length of D2 from 948 to 950 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 14.81 | 23.03 | 18.43 | 17.98 | 34.6 | 37.4 | 230.8 | -8.82 | 142.700- 146.080 |
|  |  | Aluminum | 14.74 | 22.98 | 18.41 | 17.97 | 34.4 | 34.8 | 232.1 | -8.92 |  |
|  | Vertical part | No losss | 14.80 | 24.39 | 19.63 | 19.66 | 37.6 | 35.0 | 228.5 | -8.79 | 142.600 – 146.100 |
|  |  | Aluminum | 14.72 | 24.31 | 19.62 | 19.62 | 37.6 | 35.0 | 229.8 | -8.89 |  |