ANTENNA QY21106XL2D4

Classic folded dipole as radiator. Not recommended for XPOL. If stacking wanted, please, choose the antenna model QY21106XL4D4 instead

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21106XL2D4 | 11 | 6305 | 15.49 | 22.51 | 19.18 | 18.44 | 32.4 | 34.7 | 227.4 | -8.08 | 4100 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 15.41 | 229.0 | -8.19 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21106XL2D4 | 6305 | No loss | 15.49 | 227.4 | -8.08 | 142.200- 146.300 |
|  |  | Aluminum | 15.41 | 229.0 | -8.19 |  |
| EF0211-05 | 6325 | No loss | 15.24 | 226.7 | -8.32 | 142.600-144.800 |
|  |  | Aluminum | 15.14 | 228.6 | -8.45 |  |

DIMENSIONS:

|  | Ref | De1 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | REMARKS: - All elements made of Al rods Ǿ6 - Height of all rectangles 450 mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 405 | 531 | 895 | 1489 | 2205 | 2975 | 3816 | 4698 | 5529 | 6305 |  |
| Length | 1012 | 954 | 944 | 944 | 926 | 906 | 892 | 544 | 532 | 528 | 510 |  |

Building hints:

- No boom correction included.

- Radiator is in classic, folded shape, dimension 954+40+(2x472) mm, with a gap 10 mm included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21106XL4D4 - STACKING TABLE (including loss for aluminum – AGT = -0.08 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 16.18 | 236.6 | -7.56 | 100 |  |  |  |
| 110 | 16.38 | 229.8 | -7.23 | 110 |  |  |  |
| 120 | 16.52 | 227.1 | -7.04 | 120 | 15.41 | 229.0 | -8.19 |
| 130 | 16.62 | 229.6 | -6.99 | 130 | 16.18 | 248.6 | -7.77 |
| 140 | 16.68 | 235.2 | -7.03 | 140 | 16.43 | 247.0 | -7.50 |
| 150 | 16.76 | 237.7 | -7.00 | 150 | 16.48 | 232.6 | -7.19 |
| 160 | 16.87 | 235.1 | -6.84 | 160 | 16.61 | 244.6 | -7.27 |
| 170 | 16.99 | 231.6 | -6.66 | 170 | 16.88 | 239.9 | -6.92 |
| 180 | 17.10 | 229.3 | -6.50 | 180 | 17.00 | 238.1 | -6.77 |
| 190 | 17.20 | 228.4 | -6.39 | 190 | 17.10 | 236.4 | -6.64 |
| 200 | 17.30 | 228.5 | -6.29 | 200 | 17.20 | 234.9 | -6.51 |
| 210 | 17.39 | 229.1 | -6.21 | 210 | 17.29 | 233.6 | -6.39 |
| 220 | 17.47 | 229.9 | -6.15 | 220 | 17.38 | 232.3 | -6.28 |
| 230 | 17.55 | 230.5 | -6.08 | 230 | 17.46 | 231.2 | -6.18 |
| 240 | 17.63 | 230.6 | -6.00 | 240 | 17.54 | 230.2 | -6.08 |
| 250 | 17.71 | 230.2 | -5.91 | 250 | 17.61 | 229.2 | -5.99 |
| 260 | 17.79 | 221.3 | -5.79 | 260 | 17.69 | 228.4 | -5.90 |
| 270 | 17.88 | 228.2 | 5.70 | 270 | 17.76 | 227.7 | -5.81 |
| 280 | 17.96 | 227.0 | -5.60 | 280 | 17.84 | 227.1 | -5.72 |
| 290 | 18.04 | 225.9 | -5.50 | 290 | 17.91 | 226.6 | -5.64 |
| 300 | 18.11 | 225.1 | -5.41 | 300 | 17.98 | 226.3 | -5.57 |
| 310 | 18.18 | 224.7 | -5.34 | 310 | 18.04 | 226.0 | -5.50 |
| 320 | 18.24 | 224.7 | -5.28 | 320 | 18.10 | 225.9 | -5.44 |
| 330 | 18.29 | 225.2 | -5.24 | 330 | 18.16 | 225.9 | -5.38 |
| 340 | 18.33 | 226.3 | -5.22 | 340 | 18.21 | 226.0 | -5.33 |
| 350 | 18.36 | 227.8 | -5.22 | 350 | 18.25 | 226.1 | -5.29 |
| 360 | 18.38 | 229.6 | -5.23 | 360 | 18.29 | 226.3 | -5.26 |
| 370 | 18.39 | 231.0 | -5.24 | 370 | 18.33 | 226.6 | -5.22 |
| 380 | 18.41 | 231.8 | -5.24 | 380 | 18.36 | 226.9 | -5.20 |
| 390 | 18.43 | 232.3 | -5.24 | 390 | 18.38 | 227.2 | -5.18 |
| 400 |  |  |  | 400 | 18.40 | 227.6 | -5.17 |
|  |  |  |  | 410 | 18.42 | 227.9 | -5.16 |
|  |  |  |  | 420 | 18.43 | 228.3 | -5.15 |

TWO ANTENNAS VERTICALLY STACKED AT 340CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21106XL2D4 | No losss | 18.41 | 22.49 | 20.79 | 12.09 | 32.6 | 15.6 | 224.7 | -5.11 | 142.200- 146.300 |
|  | Aluminum | 18.33 | 22.36 | 20.78 | 12.12 | 32.6 | 15.6 | 226.3 | -5.22 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 340+340+340CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21106XL2D4 | No losss | 21.37 | 23.02 | 21.85 | 8.13 | 32.8 | 7.2 | 219.7 | -2.05 | 142.200- 146.300 |
|  | Aluminum | 21.28 | 22.88 | 21.82 | 8.13 | 32.8 | 7.2 | 221.3 | -2.17 |  |

FOUR ANTENNAS IN H- STACK, H=420 CM, V=340 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21106XL2D4 | No losss | 21.48 | 22.64 | 8.66 | 11.87 | 13.0 | 15.6 | 225.5 | -2.05 | 142.200- 146.300 |
|  | Aluminum | 21.39 | 22.50 | 8.68 | 11.91 | 13.0 | 15.6 | 226.9 | -2.17 |  |