ANTENNA QY21106XL4D4

Good stacking characteristics at any vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21106XL4D4 | 11 | 6000 | 15.30 | 29.70 | 20.45 | 20.23 | 33.4 | 35.8 | 221.6 | -8.16 | 3600 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 15.20 | 223.5 | -8.29 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21106XL4D4 | 6000 | No loss | 15.35 | 221.6 | -8.16 | 142.800- 146.400 |
|  |  | Aluminum | 15.20 | 223.5 | -8.29 |  |
| EF0211-5B | 5980 | No loss | 15.13 | 229.1 | -8.47 | 140.500-145.200 |
|  |  | Aluminum | 15.04 | 230.8 | -8.59 |  |
| EF0211-05 | 6325 | No loss | 15.24 | 226.7 | -8.32 | 142.600-144.800 |
|  |  | Aluminum | 15.14 | 228.6 | -8.45 |  |

DIMENSIONS:

|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | REMARKS: - All elements made of Al rods Ǿ6 - Height of all rectangles 500 mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 200 | 350 | 428 | 722 | 1278 | 1950 | 2715 | 3525 | 4355 | 5268 | 6000 |  |
| Length | 1016 | 996 | 896 | 956 | 956 | 930 | 914 | 896 | 518 | 494 | 490 | 484 |  |

Building hints: No boom correction included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21106XL4D4 - STACKING TABLE (including loss for aluminum – AGT = -0.12 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 16.02 | 229.1 | -7.58 | 100 |  |  |  |
| 110 | 16.21 | 224.1 | -7.29 | 110 |  |  |  |
| 120 | 16.36 | 221.3 | -7.09 | 120 | 15.64 | 246.1 | -8.27 |
| 130 | 16.49 | 219.8 | -6.93 | 130 | 16.04 | 240.5 | -7.77 |
| 140 | 16.61 | 218.5 | -6.78 | 140 | 16.30 | 236.0 | -7.43 |
| 150 | 16.72 | 217.1 | -6.65 | 150 | 16.48 | 232.6 | -7.19 |
| 160 | 16.83 | 216.6 | -6.53 | 160 | 16.63 | 230.1 | -6.99 |
| 170 | 16.92 | 217.2 | -6.45 | 170 | 16.76 | 228.2 | -6.82 |
| 180 | 17.01 | 218.3 | -6.38 | 180 | 16.87 | 226.8 | -6.69 |
| 190 | 17.09 | 219.4 | -6.32 | 190 | 16.98 | 225.7 | -6.56 |
| 200 | 17.18 | 220.5 | -6.25 | 200 | 17.07 | 224.8 | -6.45 |
| 210 | 17.26 | 221.4 | -6.19 | 210 | 17.16 | 224.0 | -6.34 |
| 220 | 17.34 | 222.0 | -6.12 | 220 | 17.25 | 223.5 | -6.24 |
| 230 | 17.42 | 222.4 | -6.05 | 230 | 17.33 | 222.9 | -6.15 |
| 240 | 17.50 | 222.3 | -5.97 | 240 | 17.41 | 222.5 | -6.06 |
| 250 | 17.58 | 221.9 | -5.88 | 250 | 17.49 | 222.1 | -5.98 |
| 260 | 17.66 | 221.3 | -5.79 | 260 | 17.56 | 221.8 | -5.90 |
| 270 | 17.74 | 220.7 | -5.70 | 270 | 17.64 | 221.6 | -5.81 |
| 280 | 17.82 | 220.2 | -5.61 | 280 | 17.71 | 221.4 | -5.74 |
| 290 | 17.89 | 219.9 | -5.53 | 290 | 17.78 | 221.2 | -5.67 |
| 300 | 17.96 | 219.9 | -5.46 | 300 | 17.85 | 221.1 | -5.60 |
| 310 | 18.02 | 220.2 | -5.41 | 310 | 17.91 | 221.0 | -5.53 |
| 320 | 18.07 | 220.7 | -5.37 | 320 | 17.96 | 221.0 | -5.48 |
| 330 | 18.12 | 221.4 | -5.33 | 330 | 18.01 | 221.1 | -5.44 |
| 340 | 18.16 | 222.3 | -5.31 | 340 | 18.06 | 221.2 | -5.39 |
| 350 | 18.19 | 223.3 | -5.30 | 350 | 18.10 | 221.4 | -5.35 |
| 360 | 18.21 | 224.2 | -5.29 | 360 | 18.13 | 221.6 | -5.33 |
| 370 | 18.24 | 225.2 | -5.28 | 370 | 18.16 | 221.9 | -5.30 |
| 380 | 18.25 | 226.1 | -5.29 | 380 | 18.19 | 222.2 | -5.29 |
| 390 | 18.26 | 227.0 | -5.30 | 390 | 18.20 | 222.5 | -5.27 |
| 400 |  |  |  | 400 | 18.22 | 222.8 | -5.26 |
|  |  |  |  | 410 | 18.23 | 223.1 | -5.26 |
|  |  |  |  | 420 | 18.24 | 223.5 | -5.25 |

TWO ANTENNAS VERTICALLY STACKED AT 370CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21106XL4D4 | No losss | 18.34 | 32.40 | 22.82 | 9.65 | 33.8 | 14.6 | 223.4 | -5.15 | 142.800- 146.400 |
|  | Aluminum | 18.24 | 32.34 | 22.84 | 9.67 | 33.6 | 14.6 | 225.2 | -5.28 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 370+370+370CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21106XL4D4 | No losss | 21.34 | 32.69 | 21.62 | 12.66 | 34.0 | 6.8 | 227.2 | -2.20 | 142.800- 146.400 |
|  | Aluminum | 21.26 | 32.54 | 21.57 | 12.66 | 33.8 | 6.8 | 228.8 | -2.33 |  |

FOUR ANTENNAS IN H- STACK, H=410 CM, V=360 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21106XL3D3 | No losss | 21.36 | 31.89 | 8.41 | 10.02 | 13.2 | 15.0 | 223.6 | -2.14 | 142.800- 146.400 |
|  | Aluminum | 21.26 | 31.78 | 8.43 | 10.05 | 13.2 | 15.0 | 225.4 | -2.27 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of D1 from 956 to 954 mm

- Change length of DE2 from 896 to 886 mm

- Change position of D1 for -1 mm

In this case the SWR diagram will be like this:

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 15.30 | 29.70 | 20.45 | 20.23 | 33.4 | 35.8 | 221.6 | -8.16 | 142.800- 146.400 |
|  |  | Aluminum | 15.20 | 29.69 | 20.40 | 20.23 | 33.4 | 35.8 | 223.5 | -8.29 |  |
|  | Vertical part | No losss | 15.33 | 29.04 | 19.58 | 20.86 | 35.6 | 33.2 | 222.8 | -8.15 | 142.800 – 146.350 |
|  |  | Aluminum | 15.21 | 29.05 | 19.55 | 20.81 | 35.6 | 33.2 | 224.6 | -8.28 |  |