ANTENNA QY21108XL4D4

Good stacking characteristics at any vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21108XL4D4 | 11 | 6000 | 15.39 | 23.96 | 18.24 | 17.94 | 32.8 | 34.8 | 225.8 | -8.15 | 2900 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included for aluminum) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 15.31 | 227.2 | -8.25 |

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21108XL4D4 | 6000 | No loss | 15.39 | 225.8 | -8.15 | 143.400- 146.300 |
|  |  | Aluminum | 15.31 | 227.2 | -8.25 |  |
| EF0211-5B | 5980 | No loss | 15.13 | 229.1 | -8.47 | 140.500-145.200 |
|  |  | Aluminum | 15.04 | 230.8 | -8.59 |  |
| EF0211-05 | 6325 | No loss | 15.24 | 226.7 | -8.32 | 142.600-144.800 |
|  |  | Aluminum | 15.14 | 228.6 | -8.45 |  |

DIMENSIONS:

|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | REMARKS: - All elements made of Al rods Ǿ8 - Height of all rectangles 500 mm - All dimensions given in milimetres |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 190 | 330 | 406 | 704 | 1255 | 1943 | 2705 | 3525 | 4335 | 5203 | 6000 |  |
| Length | 1016 | 1000 | 868 | 942 | 948 | 924 | 904 | 886 | 526 | 502 | 496 | 476 |  |

Building hints: No boom correction included.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21106XL4D4 - STACKING TABLE (including loss for aluminum – AGT = -0.09 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 15.89 | 238.5 | -7.89 | 100 |  |  |  |
| 110 | 16.13 | 231.7 | -7.52 | 110 |  |  |  |
| 120 | 16.31 | 227.7 | -7.26 | 120 | 15.53 | 251.2 | -8.47 |
| 130 | 16.46 | 225.5 | -7.07 | 130 | 16.00 | 246.3 | -7.91 |
| 140 | 16.60 | 223.6 | -6.89 | 140 | 16.29 | 241.7 | -7.54 |
| 150 | 16.73 | 221.1 | -6.72 | 150 | 16.50 | 238.1 | -7.27 |
| 160 | 16.85 | 218.9 | -6.55 | 160 | 16.66 | 235.5 | -7.06 |
| 170 | 16.96 | 217.8 | -6.42 | 170 | 16.80 | 233.5 | -6.88 |
| 180 | 17.06 | 217.9 | -6.32 | 180 | 16.93 | 232.0 | -6.72 |
| 190 | 17.16 | 218.7 | -6.24 | 190 | 17.04 | 230.8 | -6.59 |
| 200 | 17.25 | 220.0 | -6.17 | 200 | 17.14 | 229.8 | -6.47 |
| 210 | 17.34 | 221.7 | -6.12 | 210 | 17.23 | 229.0 | -6.37 |
| 220 | 17.42 | 223.3 | -6.07 | 220 | 17.32 | 228.2 | -6.26 |
| 230 | 17.50 | 224.5 | -6.01 | 230 | 17.40 | 227.5 | -6.17 |
| 240 | 17.58 | 225.3 | -5.95 | 240 | 17.48 | 226.9 | -6.08 |
| 250 | 17.66 | 225.5 | -5.87 | 250 | 17.56 | 226.3 | -5.99 |
| 260 | 17.74 | 225.4 | -5.79 | 260 | 17.64 | 225.8 | -5.90 |
| 270 | 17.82 | 225.2 | -5.70 | 270 | 17.71 | 225.3 | -5.82 |
| 280 | 17.90 | 224.9 | -5.62 | 280 | 17.79 | 224.8 | -5.73 |
| 290 | 17.97 | 224.8 | -5.55 | 290 | 17.86 | 224.5 | -5.65 |
| 300 | 18.04 | 224.8 | -5.48 | 300 | 17.92 | 224.2 | -5.59 |
| 310 | 18.10 | 225.2 | -5.43 | 310 | 17.99 | 224.0 | -5.51 |
| 320 | 18.15 | 225.7 | -5.39 | 320 | 18.04 | 224.0 | -5.46 |
| 330 | 18.20 | 226.5 | -5.35 | 330 | 18.10 | 223.9 | -5.40 |
| 340 | 18.24 | 227.4 | -5.33 | 340 | 18.14 | 224.0 | -5.36 |
| 350 | 18.27 | 228.3 | -5.32 | 350 | 18.18 | 224.2 | -5.33 |
| 360 | 18.30 | 229.2 | -5.30 | 360 | 18.22 | 224.4 | -5.29 |
| 370 | 18.32 | 230.1 | -5.30 | 370 | 18.25 | 224.7 | -5.27 |
| 380 | 18.34 | 230.7 | -5.29 | 380 | 18.27 | 225.1 | -5.25 |
| 390 | 18.35 | 231.2 | -5.29 | 390 | 18.30 | 225.5 | -5.24 |
| 400 | 18.36 | 231.5 | -5.28 | 400 | 18.31 | 225.9 | -5.23 |
|  |  |  |  | 410 | 18.33 | 226.3 | -5.22 |
|  |  |  |  | 420 | 18.34 | 226.7 | -5.21 |

TWO ANTENNAS VERTICALLY STACKED AT 380CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21108XL4D4 | No losss | 18.42 | 25.53 | 18.90 | 9.47 | 32.9 | 14.2 | 229.4 | -5.19 | 143.400- 146.300 |
|  | Aluminum | 18.34 | 25.56 | 18.87 | 9.49 | 32.8 | 14.2 | 230.7 | -5.29 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 380+380+380CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21110XL4D4 | No losss | 21.44 | 26.23 | 19.67 | 12.86 | 33.2 | 6.6 | 234.3 | -2.26 | 143.400- 146.300 |
|  | Aluminum | 21.36 | 26.25 | 19.67 | 12.86 | 33.0 | 6.6 | 235.5 | -2.36 |  |

FOUR ANTENNAS IN H- STACK, H=420 CM, V=380 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21108XL4D4 | No losss | 21.43 | 25.49 | 9.18 | 9.67 | 13.4 | 14.4 | 229.5 | -2.18 | 143.400- 146.300 |
|  | Aluminum | 21.35 | 25.50 | 9.19 | 9.69 | 13.4 | 14.4 | 230.8 | -2.28 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change position of D1 for -3 mm

- Change length of D2 from 948 to 949 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 15.39 | 23.96 | 18.24 | 17.94 | 32.8 | 34.8 | 225.8 | -8.15 | 143.400- 146.300 |
|  |  | Aluminum | 15.31 | 23.98 | 18.21 | 17.93 | 32.6 | 34.8 | 227.2 | -8.25 |  |
|  | Vertical part | No losss | 15.40 | 24.56 | 18.84 | 19.39 | 34.8 | 32.8 | 226.4 | -8.15 | 143.300 – 146.300 |
|  |  | Aluminum | 15.32 | 24.62 | 18.84 | 19.36 | 34.8 | 32.8 | 227.8 | -8.26 |  |