ANTENNA QY21205XL4D5

Good stacking characteristics at big vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21205XL4D5 | 12 | 6838 | 15.77 | 28.75 | 18.93 | 18.04 | 31.8 | 33.6 | 219.6 | -7.65 | 3000 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Losses included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 15.66 | 221.7 | -7.80 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | DQ5 | REMARKS : - All elements made of Al rods Ǿ5 mm - The height of the rectangle is 500 mm - All dimensions given in milimetres |
| Pos | 0 | 265 | 345 | 415 | 705 | 1252 | 1934 | 2695 | 3535 | 4355 | 5233 | 6138 | 6838 |  |
| Length | 1018 | 1018 | 830 | 946 | 958 | 932 | 916 | 900 | 534 | 502 | 492 | 482 | 468 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21205XL4D5 | 6838 | No loss | 15.77 | 219.6 | -7.65 | 143.100- 146.100 |
|  |  | Aluminum | 15.66 | 221.7 | -7.80 |  |
| G0KSC 12ELKKA | 6906 | No loss | 15.69 | 222.4 | -7.78 | 142.600- 145.450 |
|  |  | Aluminum | 15.61 | 223.8 | -7.89 |  |
| EF0212-5 | 7260 | No loss | 15.91 | 224.0 | -7.59 | 143.300-144.820 |
|  |  | Aluminum | 15.81 | 226.0 | -7.73 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21205XL4D5 - STACKING TABLE (including loss for aluminum – AGT = -0.10 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 16.24 | 235.9 | -7.49 | 100 |  |  |  |
| 110 | 16.49 | 228.7 | -7.10 | 110 | 15.06 | 257.3 | -9.04 |
| 120 | 16.67 | 224.7 | -6.85 | 120 | 15.99 | 250.1 | -7.99 |
| 130 | 16.81 | 222.5 | -6.66 | 130 | 16.41 | 243.1 | -7.45 |
| 140 | 16.94 | 220.7 | -6.50 | 140 | 16.68 | 237.7 | -7.08 |
| 150 | 17.06 | 218.3 | -6.33 | 150 | 16.87 | 233.9 | -6.82 |
| 160 | 17.17 | 217.0 | -6.20 | 160 | 17.02 | 231.4 | -6.62 |
| 170 | 17.26 | 216.6 | -6.10 | 170 | 17.14 | 229.6 | -6.47 |
| 180 | 17.35 | 216.9 | -6.01 | 180 | 17.25 | 228.3 | -6.33 |
| 190 | 17.44 | 217.7 | -5.94 | 190 | 17.35 | 227.2 | -6.21 |
| 200 | 17.52 | 218.9 | -5.88 | 200 | 17.44 | 226.3 | -6.11 |
| 210 | 17.60 | 220.2 | -5.83 | 210 | 17.53 | 225.5 | -6.00 |
| 220 | 17.67 | 221.6 | -5.78 | 220 | 17.61 | 224.7 | -5.91 |
| 230 | 17.75 | 222.8 | -5.73 | 230 | 17.68 | 223.9 | -5.82 |
| 240 | 17.83 | 223.6 | -5.66 | 240 | 17.76 | 223.2 | -5.73 |
| 250 | 17.90 | 223.9 | -5.60 | 250 | 17.83 | 222.4 | -5.64 |
| 260 | 17.98 | 223.7 | -5.52 | 260 | 17.91 | 221.7 | -5.55 |
| 270 | 18.06 | 223.1 | -5.42 | 270 | 17.98 | 221.1 | -5.47 |
| 280 | 18.14 | 222.3 | -5.33 | 280 | 18.05 | 220.5 | -5.38 |
| 290 | 18.22 | 221.5 | -5.23 | 290 | 18.12 | 220.0 | -5.30 |
| 300 | 18.29 | 220.7 | -5.15 | 300 | 18.19 | 219.6 | -5.23 |
| 310 | 18.36 | 220.2 | -5.07 | 310 | 18.26 | 219.2 | -5.15 |
| 320 | 18.42 | 219.8 | -5.00 | 320 | 18.32 | 219.0 | -5.08 |
| 330 | 18.48 | 219.8 | -4.94 | 330 | 18.31 | 218.9 | -5.02 |
| 340 | 18.53 | 220.0 | -4.89 | 340 | 18.43 | 218.9 | -4.97 |
| 350 | 18.57 | 220.3 | -4.86 | 350 | 18.48 | 218.9 | -4.92 |
| 360 | 18.61 | 220.8 | -4.83 | 360 | 18.52 | 219.1 | -4.89 |
| 370 | 18.64 | 221.4 | -4.81 | 370 | 18.56 | 219.3 | -4.85 |
| 380 | 18.66 | 221.9 | -4.80 | 380 | 18,59 | 219.6 | -4.83 |
| 390 | 18.68 | 222.3 | -4.79 | 390 | 18.62 | 219.9 | -4.80 |
| 400 | 18.70 | 222.8 | -4.78 | 400 | 18.64 | 220.3 | -4.79 |
| 410 | 18.71 | 223.1 | -4.77 | 410 | 18.66 | 220.7 | -4.78 |
| 420 | 18.72 | 223.2 | -4.77 | 420 | 18.68 | 221.0 | -4.76 |
|  |  |  |  | 430 | 18.69 | 221.4 | -4.76 |
|  |  |  |  | 440 | 18.70 | 221.6 | -4.76 |
|  |  |  |  | 450 | 18.71 | 221.9 | -4.75 |

Remark: brown colour letters show not consistent data!

TWO ANTENNAS VERTICALLY STACKED AT 410CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21205XL4D5 | No losss | 18.82 | 28.91 | 19.61 | 8.74 | 32.2 | 13.2 | 221.0 | -4.62 | 143.100- 146.100 |
|  | Aluminum | 18.71 | 28.64 | 19.57 | 8.77 | 32.0 | 13.2 | 223.0 | -4.77 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 400+400+400CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21205XL4D5 | No losss | 21.85 | 28.27 | 20.06 | 13.16 | 32.2 | 6.4 | 223.8 | -1.65 | 143.100- 146.100 |
|  | Aluminum | 21.73 | 27.99 | 20.06 | 13.20 | 32.2 | 6.4 | 225.8 | -1.81 |  |

FOUR ANTENNAS IN H- STACK, H=459 CM, V=405 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21205XL4D5 | No losss | 21.87 | 28.65 | 7.42 | 8.86 | 12.2 | 13.4 | 222.7 | -1.61 | 143.100- 146.100 |
|  | Aluminum | 21.76 | 28.38 | 7.44 | 8.89 | 12.2 | 13.4 | 224.7 | -1.76 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE2 from 830 to 898 mm

- Change length of D1 from 946 to 950 mm

- Change length of D2 from 958 to 956 mm

- Change length of D3 from 932 to 928 mm

- Change length of D5 from 900 to 948 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 15.77 | 28.75 | 18.93 | 18.04 | 31.8 | 33.6 | 219.6 | -7.65 | 143.100- 146.100 |
|  |  | Aluminum | 15.66 | 28.49 | 18.89 | 18.02 | 31.8 | 33.6 | 221.7 | -7.80 |  |
|  | Vertical part | No losss | 15.82 | 42.17 | 17.01 | 18.76 | 33.4 | 31.8 | 220.0 | -7.60 | 142.100 – 145.900 |
|  |  | Aluminum | 15.72 | 40.81 | 17.00 | 18.73 | 33.4 | 31.6 | 222.0 | -7.74 |  |