ANTENNA QY21206XL4D5

Good stacking characteristics at any vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21206XL4D5 | 12 | 6885 | 15.84 | 30.27 | 28.71 | 18.28 | 31.4 | 32.8 | 221.6 | -7.62 | 2810 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 15.75 | 223.3 | -7.74 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | DQ5 | REMARKS : All elements made of Al rods Ǿ8 mm The height of the rectangle is 500 mm All dimensions given in milimetres |
| Pos | 0 | 180 | 330 | 409 | 708 | 1254 | 1938 | 2705 | 3525 | 4355 | 5233 | 6108 | 6885 |  |
| Length | 1012 | 990 | 866 | 942 | 951 | 925 | 907 | 887 | 526 | 504 | 496 | 486 | 466 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21206XL4D5 | 6885 | No loss | 15.84 | 221.6 | -7.62 | 143.210- 146.020 |
|  |  | Aluminum | 15.75 | 223.3 | -7.74 |  |
| G0KSC 12ELKKA | 6906 | No loss | 15.69 | 222.4 | -7.78 | 142.600-145.450 |
|  |  | Aluminum | 15.61 | 223.8 | -7.89 |  |
| EF0212-5 | 7260 | No loss | 15.91 | 224.0 | -7.59 | 143.300-144.820 |
|  |  | Aluminum | 15.81 | 226.0 | -7.73 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21208XL4D5 - STACKING TABLE (including loss for aluminum – AGT = -0.09 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 16.08 | 238.4 | -7.69 | 100 |  |  |  |
| 110 | 16.37 | 231.1 | -7.27 | 110 |  |  |  |
| 120 | 16.59 | 227.3 | -6.98 | 120 | 15.73 | 246.9 | -8.20 |
| 130 | 16.76 | 225.5 | -6.77 | 130 | 16.28 | 241.9 | -7.56 |
| 140 | 16.90 | 223.7 | -6.60 | 140 | 16.59 | 238.1 | -7.18 |
| 150 | 17.05 | 221.0 | -6.39 | 150 | 16.82 | 235.0 | -6.89 |
| 160 | 17.18 | 218.2 | -6.21 | 160 | 17.00 | 232.6 | -6.67 |
| 170 | 17.30 | 216.6 | -6.06 | 170 | 17.14 | 230.8 | -6.49 |
| 180 | 17.41 | 216.1 | -5.94 | 180 | 17.27 | 229.4 | -6.34 |
| 190 | 17.51 | 216.5 | -5.84 | 190 | 17.39 | 228.3 | -6.19 |
| 200 | 17.60 | 217.4 | -5.77 | 200 | 17.49 | 227.3 | -6.08 |
| 210 | 17.69 | 218.7 | -5.71 | 210 | 17.59 | 226.5 | -5.96 |
| 220 | 17.77 | 220.2 | -5.66 | 220 | 17.68 | 225.7 | -5.86 |
| 230 | 17.85 | 221.7 | -5.61 | 230 | 17.76 | 225.1 | -5.76 |
| 240 | 17.92 | 222.8 | -5.56 | 240 | 17.84 | 224.4 | -5.67 |
| 250 | 18.00 | 223.5 | -5.49 | 250 | 17.92 | 223.7 | -5.58 |
| 260 | 18.08 | 223.7 | -5.42 | 260 | 18.00 | 223.1 | -5.49 |
| 270 | 18.15 | 223.5 | -5.34 | 270 | 18.08 | 222.5 | -5.39 |
| 280 | 18.23 | 223.1 | -5.25 | 280 | 18.15 | 222.0 | -5.31 |
| 290 | 18.30 | 222.6 | -5.18 | 290 | 18.22 | 221.5 | -5.23 |
| 300 | 18.37 | 222.2 | -5.10 | 300 | 18.29 | 221.0 | -5.15 |
| 310 | 18.44 | 221.9 | -5.02 | 310 | 18.35 | 220.6 | -5.09 |
| 320 | 18.50 | 221.8 | -4.96 | 320 | 18.41 | 220.4 | -5.02 |
| 330 | 18,55 | 222.0 | -4.91 | 330 | 18.47 | 220.2 | -4.96 |
| 340 | 18.60 | 222.4 | -4.87 | 340 | 18.52 | 220.2 | -4.91 |
| 350 | 18.64 | 222.9 | -4.84 | 350 | 18.56 | 220.2 | -4.87 |
| 360 | 18.67 | 223.4 | -4.82 | 360 | 18.60 | 220.4 | -4.83 |
| 370 | 18.71 | 223.9 | -4.79 | 370 | 18.64 | 220.5 | -4.80 |
| 380 | 18.73 | 224.2 | -4.78 | 380 | 18.67 | 220.8 | -4.77 |
| 390 | 18.75 | 224.4 | -4.76 | 390 | 18.69 | 221.1 | -4.76 |
| 400 | 18.77 | 224.6 | -4.74 | 400 | 18.71 | 221.4 | -4.74 |
| 410 | 18.79 | 224.7 | -4.73 | 410 | 18.73 | 221.7 | -4.73 |
| 420 | 18.80 | 224.7 | -4.72 | 420 | 18.75 | 222.1 | -4.71 |
| 430 | 18.80 | 224.6 | -4.71 | 430 | 18.76 | 222.4 | -4.71 |
| 440 |  |  |  | 440 | 18.77 | 222.6 | -4.71 |
| 450 |  |  |  | 450 |  |  |  |
| 460 |  |  |  | 460 |  |  |  |
| 470 |  |  |  | 470 |  |  |  |
| 480 |  |  |  | 480 |  |  |  |
| 490 |  |  |  | 490 |  |  |  |
| 500 |  |  |  | 500 |  |  |  |
| 510 |  |  |  | 510 |  |  |  |
| 520 |  |  |  | 520 |  |  |  |
| 530 |  |  |  | 530 |  |  |  |
| 540 |  |  |  | 540 |  |  |  |
| 550 |  |  |  | 550 |  |  |  |

Remark: brown colour letters show not consistent data!

TWO ANTENNAS VERTICALLY STACKED AT 420CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21208XL4D5 | No losss | 18.88 | 32.19 | 19.52 | 8.71 | 31.6 | 13.0 | 223.1 | -4.60 | 143.210- 146.020 |
|  | Aluminum | 18.80 | 32.13 | 19.52 | 8.74 | 31.4 | 13.0 | 224.7 | -4.72 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 420+420+420 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21208XL4D5 | No losss | 21.92 | 33.81 | 19.61 | 11.37 | 31.6 | 6.2 | 227.0 | -1.64 | 143.210- 146.020 |
|  | Aluminum | 21.83 | 34.06 | 19.60 | 11.39 | 31.6 | 6.1 | 228.5 | -1.76 |  |

FOUR ANTENNAS IN H- STACK, H=420 CM, V=420 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21208XL4D5 | No losss | 21.89 | 32.71 | 9.23 | 8.64 | 12.8 | 13.0 | 223.1 | -1.60 | 143.210- 146.020 |
|  | Aluminum | 21.81 | 32.61 | 9.26 | 8.67 | 12.8 | 13.0 | 224.7 | -1.71 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of D1 from 942 to 945 mm

- Change length of D2 from 951 to 952 mm

- Change length of D3 from 925 to 923 mm

- Change length of D4 from 907 to 906 mm

- Change length of D5 from 887 to 886 mm

- Change position od D1 for +1 mm.

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 15.84 | 30.27 | 28.71 | 18.28 | 31.4 | 32.8 | 221.6 | -7.62 | 143.210- 146.020 |
|  |  | Aluminum | 15.75 | 30.20 | 28.67 | 18.28 | 31.2 | 32.8 | 223.3 | -7.74 |  |
|  | Vertical part | No losss | 15.88 | 31.19 | 17.93 | 19.57 | 32.6 | 31.2 | 223.0 | -7.60 | 143.200 – 146.300 |
|  |  | Aluminum | 15.79 | 31.40 | 17.94 | 19.54 | 32.6 | 31.2 | 224.6 | -7.72 |  |