ANTENNA QY21310XL4D6

Good stacking characteristics at big vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21310XL4D6 | 13 | 7775 | 16.28 | 39.07 | 19.17 | 17.59 | 30.2 | 31.2 | 220.8 | -7.16 | 3000 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 16.20 | 222.2 | -7.27 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | DQ5 | DQ6 | REMARKS : - All elements made of Al rods Ǿ10 mm - The height of the rectangle is 500 mm - All dimensions given in milimetres |
| Pos | 0 | 215 | 335 | 415 | 709 | 1258 | 1939 | 2705 | 3525 | 4358 | 5239 | 6138 | 6999 | 7775 |  |
| Length | 1010 | 990 | 910 | 940 | 946 | 918 | 898 | 878 | 524 | 502 | 498 | 486 | 478 | 466 |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21310XL4D6 | 7775 | No loss | 16.28 | 220.8 | -7.16 | 143.250- 146.250 |
|  |  | Aluminum | 16.20 | 222.2 | -7.27 |  |
| G0KSC 13ELKKB | 7855 | No loss | 16.13 | 219.4 | -7.28 | 142.650-145.370 |
|  |  | Aluminum | 16.03 | 221.1 | -7.42 |  |
| EF0213-5 | 8150 | No loss | 16.40 | 224.8 | -7.12 | 143.660-144.620 |
|  |  | Aluminum | 16.25 | 227.6 | -7.32 |  |

DIAGRAMS (no loss included)

DIAGRAMS (losses included)

STACKING

| QY21310XL4D6 (including loss for aluminum – AGT = -0.08 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 16.37 | 240.5 | -7.44 | 100 |  |  |  |
| 110 | 16.70 | 231.9 | -6.95 | 110 |  |  |  |
| 120 | 16.94 | 226.8 | -6.62 | 120 | 16.20 | 249.6 | -7.77 |
| 130 | 17.13 | 223.6 | -6.37 | 130 | 16.71 | 241.8 | -7.12 |
| 140 | 17.29 | 220.9 | -6.15 | 140 | 17.01 | 238.3 | -6.74 |
| 150 | 17.44 | 218.0 | -5.94 | 150 | 17.22 | 234.0 | -6.47 |
| 160 | 17.57 | 215.6 | -5.77 | 160 | 17.39 | 231.6 | -6.26 |
| 170 | 17.68 | 214.5 | -5.63 | 170 | 17.53 | 230.0 | -6.09 |
| 180 | 17.79 | 214.4 | -5.52 | 180 | 17.66 | 228.7 | -5.93 |
| 190 | 17.88 | 215.0 | -5.44 | 190 | 17.77 | 227.8 | -5.80 |
| 200 | 17.97 | 216.1 | -5.38 | 200 | 17.87 | 227.0 | -5.69 |
| 210 | 18.05 | 217.7 | -5.33 | 210 | 17.96 | 226.4 | -5.59 |
| 220 | 18.12 | 219.6 | -5.30 | 220 | 18.05 | 225.8 | -5.49 |
| 230 | 18.19 | 221.6 | -5.27 | 230 | 18.13 | 225.2 | -5.40 |
| 240 | 18.26 | 223.5 | -5.23 | 240 | 18.21 | 224.6 | -5.30 |
| 250 | 18.33 | 224.9 | -5.19 | 250 | 18.28 | 224.0 | -5.22 |
| 260 | 18.40 | 225.8 | -5.14 | 260 | 18.36 | 223.4 | -5.13 |
| 270 | 18.47 | 226.0 | -5.07 | 270 | 18.43 | 222.6 | -5.05 |
| 280 | 18.54 | 225.7 | -4.99 | 280 | 18.50 | 222.0 | -4.96 |
| 290 | 18.62 | 225.0 | -4.90 | 290 | 18.57 | 221.3 | -4.88 |
| 300 | 18.69 | 224.2 | -4.82 | 300 | 18.64 | 220.6 | -4.80 |
| 310 | 18.76 | 223.3 | -4.73 | 310 | 18.71 | 220.1 | -4.72 |
| 320 | 18.83 | 222.5 | -4.64 | 320 | 18.77 | 219.6 | -4.65 |
| 330 | 18.89 | 221.9 | -4.57 | 330 | 18.83 | 219.3 | -4.58 |
| 340 | 18.95 | 221.5 | 4.50 | 340 | 18.89 | 219.0 | -4.51 |
| 350 | 19.00 | 221.3 | -4.45 | 350 | 18.94 | 218.9 | -4.46 |
| 360 | 19.05 | 221.2 | -4.40 | 360 | 18.99 | 218.9 | -4.41 |
| 370 | 19.09 | 221.2 | -4.36 | 370 | 19.03 | 218.9 | -4.37 |
| 380 | 19.12 | 221.2 | -4.33 | 380 | 19.07 | 219.1 | -4.34 |
| 390 | 19.16 | 221.2 | -4.29 | 390 | 19.10 | 219.3 | -4.31 |
| 400 | 19.18 | 221.1 | -4.27 | 400 | 19.13 | 219.6 | -4.29 |
| 410 | 19.21 | 221.0 | -4.23 | 410 | 19.15 | 219.9 | -4.27 |
| 420 | 19.22 | 220.9 | -4.22 | 420 | 19.17 | 220.2 | -4.26 |
| 430 | 19.24 | 220.8 | -4.20 | 430 | 19.19 | 220.5 | -4.24 |
| 440 | 19.25 | 220.6 | -4.19 | 440 | 19.20 | 220.8 | -4.24 |
| 450 | 19.26 | 220.4 | -4.17 | 450 | 19.22 | 221.0 | -4.22 |
| 460 | 19.27 | 220.2 | 4.16 | 460 | 19.23 | 221.3 | -4.22 |
| 470 | 19.27 | 220.0 | -4.15 | 470 | 19.24 | 221.5 | -4.21 |
| 480 | 19.27 | 219.7 | -4.15 | 480 | 19.24 | 221.7 | -4.22 |

Remark: brown colour letters show not consistent data!

TWO ANTENNAS VERTICALLY STACKED AT 480CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21310XL4D6 | No losss | 19.35 | 40.12 | 20.24 | 7.30 | 30.4 | 11.6 | 218.3 | -4.04 | 143.250- 146.250 |
|  | Aluminum | 19.27 | 39.66 | 20.26 | 7.31 | 30.3 | 11.6 | 219.7 | -4.15 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 480+480+480 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21310XL4D6 | No losss | 22.39 | 38.50 | 20.24 | 9.2 | 30.4 | 5.1 | 216.3 | -0.96 | 143.250- 146.250 |
|  | Aluminum | 22.31 | 38.14 | 20.25 | 9.22 | 30.4 | 5.1 | 217.6 | -1.07 |  |

FOUR ANTENNAS IN H- STACK, H=440 CM, V=480 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21310XL4D6 | No losss | 22.35 | 40.08 | 9.09 | 7.25 | 12.4 | 11.6 | 218.6 | -1.05 | 143.250- 146.250 |
|  | Aluminum | 22.27 | 39.56 | 9.10 | 7.26 | 12.4 | 11.6 | 220.0 | -1.15 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE1 from 990 to 1010 mm

- Change length of DE2 from 910 to 850 mm

- Change length of D3 from 918 to 920 mm

- Change length of D3 from 898 to 896 mm

- Change position od D1 for +3 mm.

- Change position od D2 for +10 mm.

In this case the SWR diagram will be like this:

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 16.28 | 39.07 | 19.17 | 17.59 | 30.2 | 31.2 | 220.8 | -7.16 | 143.250- 146.250 |
|  |  | Aluminum | 16.20 | 38.71 | 19.14 | 17.59 | 30.2 | 31.2 | 222.2 | -7.27 |  |
|  | Vertical part | No losss | 16.32 | 31.90 | 16.57 | 19.41 | 30.8 | 29.8 | 222.4 | -7.15 | 143.300 – 146.300 |
|  |  | Aluminum | 16.24 | 32.16 | 16.59 | 19.40 | 30.8 | 29.8 | 223.8 | -7.26 |  |