ANTENNA Y21408X3D7

Good stacking characteristics at big vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21408XL3D7 | 14 | 8655 | 16.61 | 28.32 | 19.68 | 17.45 | 29.2 | 30.4 | 218.5 | -6.78 | 4700 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 16.53 | 220.1 | -6.90 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | DQ5 | DQ6 | DQ7 | REMARKS : All elements made of Al rods Ǿ8mm The height of the rectangles 500mm All dimensions given in milimetres |
| Pos | 0 | 160 | 330 | 405 | 702 | 1258 | 1939 | 2705 | 3525 | 4355 | 5223 | 6108 | 6965 | 7895 | 8655 |  |
| Length | 1012 | 1012 | 884 | 948 | 950 | 924 | 904 | 886 | 524 | 506 | 496 | 486 | 476 | 470 | 456 |  |

Building hints: No boom correction included

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21408XL3D7 | 8655 | No loss | 16.61 | 218.5 | -6.78 | 141.600 - 146.300 |
|  |  | Aluminum | 16.53 | 220.1 | -6.90 |  |
| G0KSC 14ELJJD | 8817 | No loss | 16.55 | 220.7 | -6.89 | 142.500 – 145.300 |
|  |  | Aluminum | 16.40 | 223.2 | -7.09 |  |
| EF0214-5 | 9100 | No loss | 16.81 | 220.7 | -6.63 | 143.720 - 144.620 |
|  |  | Aluminum | 16.67 | 235.4 | -6.82 |  |

.

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21408XL3D7 – STACKING TABLE (including loss for aluminum – AGT = -0.09 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 16.64 | 232.2 | -7.02 | 100 |  |  |  |
| 110 | 16.98 | 225.7 | -6.56 | 110 |  |  |  |
| 120 | 17.23 | 221.9 | -6.23 | 120 | 16.58 | 239.1 | -7.21 |
| 130 | 17.42 | 219.5 | -5.99 | 130 | 17.04 | 233.5 | -6.64 |
| 140 | 17.58 | 217.7 | -5.80 | 140 | 17.33 | 229.8 | -6.28 |
| 150 | 17.72 | 215.9 | -5.62 | 150 | 17.54 | 227.4 | -6.03 |
| 160 | 17.84 | 214.6 | -5.48 | 160 | 17.70 | 226.0 | -5.84 |
| 170 | 17.95 | 213.9 | -5.35 | 170 | 17.83 | 225.1 | -5.69 |
| 180 | 18.05 | 213.8 | -5.25 | 180 | 17.95 | 224.6 | -5.56 |
| 190 | 18.14 | 214.0 | -5.16 | 190 | 18.05 | 224.3 | -5.46 |
| 200 | 18.23 | 214.6 | -5.09 | 200 | 18.14 | 224.0 | -5.36 |
| 210 | 18.31 | 215.5 | -5.02 | 210 | 18.23 | 223.9 | -5.27 |
| 220 | 18.39 | 216.7 | -4.97 | 220 | 18.31 | 223.7 | -5.19 |
| 230 | 18.46 | 218.2 | -4.93 | 230 | 18.39 | 223.4 | -5.10 |
| 240 | 18.53 | 219.7 | -4.89 | 240 | 18.47 | 223.1 | -5.01 |
| 250 | 18.60 | 221.1 | -4.85 | 250 | 18.54 | 222.6 | -4.94 |
| 260 | 18.66 | 222.3 | -4.81 | 260 | 18.62 | 222.1 | 4.85 |
| 270 | 18.73 | 223.0 | -4.75 | 270 | 18.69 | 221.5 | -4.76 |
| 280 | 18.80 | 223.2 | -4.69 | 280 | 18.76 | 220.8 | -4.68 |
| 290 | 18.87 | 223.0 | -4.61 | 290 | 18.83 | 220.2 | -4.60 |
| 300 | 18.94 | 222.6 | -4.53 | 300 | 18.90 | 219.5 | -4.52 |
| 310 | 19.01 | 221.9 | -4.45 | 310 | 18.96 | 219.0 | -4.44 |
| 320 | 19.07 | 221.2 | -4.38 | 320 | 19.03 | 218.4 | -4.36 |
| 330 | 19.14 | 220.6 | -4.30 | 330 | 19.09 | 218.0 | -4.30 |
| 340 | 19.20 | 220.2 | -4.23 | 340 | 19.15 | 217.7 | -4.23 |
| 350 | 19.25 | 219.9 | -4.17 | 350 | 19.20 | 217.5 | -4.17 |
| 360 | 19.30 | 219.8 | -4.12 | 360 | 19.26 | 217.4 | -4.11 |
| 370 | 19.35 | 219.7 | -4.07 | 370 | 19.30 | 217.3 | -4.07 |
| 380 | 19.39 | 219.6 | -4.03 | 380 | 19.34 | 217.4 | -4.03 |
| 390 | 19.42 | 219.6 | -4.00 | 390 | 19.38 | 217.5 | -3.99 |
| 400 | 19.46 | 219.6 | -3.96 | 400 | 19.41 | 217.7 | -3.97 |
| 410 | 19.48 | 219.6 | -3.94 | 410 | 19.44 | 217.9 | -3.94 |
| 420 | 19.51 | 219.5 | -3.90 | 420 | 19.47 | 218.1 | -3.92 |
| 430 | 19.53 | 219.4 | -3.88 | 430 | 19.49 | 218.3 | -3.90 |
| 440 | 19.54 | 219.1 | -3.87 | 440 | 19.51 | 218.5 | -3.88 |
| 450 | 19.56 | 218.8 | -3.84 | 450 | 19.52 | 218.7 | -3.88 |
| 460 | 19.57 | 218.4 | -3.82 | 460 | 19.54 | 218.8 | -3.86 |
| 470 | 19.58 | 217.9 | -3.80 | 470 | 19.55 | 219.0 | -3.85 |
| 480 | 19.59 | 217.4 | -3.78 | 480 | 19.56 | 219.1 | -3.85 |
| 490 | 19.59 | 216.9 | -3.77 | 490 | 19.57 | 219.2 | -3.84 |
| 500 | 19.60 | 216.4 | -3.75 | 500 | 19.57 | 219.3 | -3.84 |

Remark: brown colour letters show not consistent data!

TWO ANTENNAS VERTICALLY STACKED AT 500CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21308XL3D7 | No losss | 19.69 | 27.97 | 19.53 | 7.37 | 29.6 | 11.0 | 214.7 | -3.63 | 141.600 - 146.300 |
|  | Aluminum | 19.60 | 27.89 | 19.51 | 7.38 | 29.6 | 11.0 | 216.4 | -3.75 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 500+500+500 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21308XL3D7 | No losss | 22.73 | 2785 | 22.39 | 9.49 | 29.6 | 4.6 | 212.6 | -0.55 | 141.600 - 146.300 |
|  | Aluminum | 22.64 | 27.77 | 22.42 | 9.51 | 29.6 | 4.6 | 214.7 | -0.63 |  |

FOUR ANTENNAS IN H- STACK, H=500 CM, V=490 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21308XL3D7 | No losss | 22.71 | 27.64 | 7.88 | 7.29 | 11.2 | 11.0 | 214.7 | -0.63 | 141.600 - 146.300 |
|  | Aluminum | 22.63 | 27.57 | 7.89 | 7.31 | 11.2 | 11.0 | 217.0 | -0.73 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE2 from 884 to 874 mm

- Change length of D1 from 950 to 948 mm

- Change length of D3 from 924 to 916 mm

- Change length of D4 from 904 to 900 mm

- Change length of D5 from 886 to 880 mm

- Change position of D2 for +5 mm.

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 16.61 | 28.32 | 19.68 | 17.45 | 29.2 | 30.4 | 218.5 | -6.78 | 141.600 - 146.300 |
|  |  | Aluminum | 16.53 | 28.24 | 19.56 | 17.47 | 29.2 | 30.4 | 220.1 | -6.90 |  |
|  | Vertical part | No losss | 16.72 | 23.74 | 15.01 | 18.27 | 29.6 | 28.6 | 221.5 | -6.73 | 140.900 – 145.600 |
|  |  | Aluminum | 16.64 | 23.74 | 15.01 | 18.24 | 29.6 | 28.6 | 222.9 | -6.84 |  |