ANTENNA Y21608XL3D9

Good stacking characteristics at big vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21608XL3D9 | 16 | 10265 | 17.15 | 27.98 | 20.66 | 17.61 | 27.6 | 28.4 | 216.0 | -6.20 | 2950 |

* ΔF given for SWR=1.5

| PERFORMANCES: (Losses included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 17.06 | 217.8 | -6.32 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | DQ5 | DQ6 | DQ7 | DQ8 | DQ9 |
| Pos | 0 | 205 | 335 | 416 | 704 | 1252 | 1936 | 2705 | 3525 | 4355 | 5223 | 6108 | 6965 | 7865 | 8679 | 9525 | 10265 |
| Length | 1010 | 1000 | 890 | 944 | 952 | 924 | 904 | 890 | 524 | 504 | 500 | 486 | 478 | 470 | 464 | 458 | 438 |
| REMARKS : All elements made of Al rods Ǿ8 mm The height of the rectangles 500 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21608XL3D9 | 10265 | No loss | 17.15 | 216.0 | -6.20 | 143.100 - 146.050 |
|  |  | Aluminum | 17.06 | 217.8 | -6.32 |  |
| EF0215-06 | 10060 | No loss | 17.23 | 226.3 | -6.32 | 143.700 – 144.450 |
|  |  | Aluminum | 17.04 | 229.6 | -6.57 |  |
| G0KSC 16ELKKA | 10696 | No loss | 17.06 | 209.8 | -6.16 | 143.000 – 145.260 |
|  |  | Aluminum | 16.95 | 211.8 | -6.31 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21608XL3D9 - STACKING TABLE (including loss for aluminum – AGT = -0.10 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 17.05 | 228.6 | -6.40 | 100 |  |  |  |
| 110 | 17.40 | 223.4 | -6.09 | 110 |  |  |  |
| 120 | 17.66 | 220.7 | -5.78 | 120 | 16.94 | 241.7 | -6.89 |
| 130 | 17.85 | 219.4 | -5.56 | 130 | 17.43 | 234.5 | -6.27 |
| 140 | 18.01 | 218.4 | -5.38 | 140 | 17.73 | 230.4 | -5.90 |
| 150 | 18.15 | 216.7 | -5.21 | 150 | 17.95 | 227.8 | -5.62 |
| 160 | 18.27 | 215.2 | -5.06 | 160 | 18.12 | 226.0 | -5.42 |
| 170 | 18.38 | 214.6 | -4.94 | 170 | 18.26 | 224.8 | -5.26 |
| 180 | 18.47 | 214.6 | -4.85 | 180 | 18.38 | 224.0 | -5.12 |
| 190 | 18.57 | 214.9 | -4.75 | 190 | 18.48 | 223.5 | -5.01 |
| 200 | 18.65 | 215.2 | -4.68 | 200 | 18.58 | 223.1 | -4.91 |
| 210 | 18.73 | 215.7 | -4.61 | 210 | 18.66 | 222.9 | -4.82 |
| 220 | 18.81 | 216.3 | -4.54 | 220 | 18.75 | 222.7 | -4.73 |
| 230 | 18.89 | 217.0 | -4.47 | 230 | 18.82 | 222.6 | -4.65 |
| 240 | 18.96 | 217.9 | -4.42 | 240 | 18.90 | 222.3 | -4.57 |
| 250 | 19.03 | 218.9 | -4.37 | 250 | 18.97 | 222.0 | -4.49 |
| 260 | 19.09 | 219.8 | -4.33 | 260 | 19.04 | 221.6 | -4.42 |
| 270 | 19.15 | 220.6 | -4.29 | 270 | 19.11 | 221.1 | -4.34 |
| 280 | 19.22 | 221.2 | -4.23 | 280 | 19.18 | 220.5 | -4.25 |
| 290 | 19.28 | 221.3 | -4.17 | 290 | 19.24 | 219.8 | -4.18 |
| 300 | 19.34 | 221.2 | -4.11 | 300 | 19.31 | 219.1 | -4.10 |
| 310 | 19.41 | 220.8 | -4.03 | 310 | 19.38 | 218.3 | -4.01 |
| 320 | 19.47 | 220.2 | -3.96 | 320 | 19.44 | 217.6 | -3.94 |
| 330 | 19.54 | 219.6 | -3.88 | 330 | 19.50 | 217.0 | -3.86 |
| 340 | 19.60 | 219.1 | -3.81 | 340 | 19.57 | 216.4 | -3.78 |
| 350 | 19.66 | 218.6 | -3.74 | 350 | 19.62 | 216.0 | -3.72 |
| 360 | 19.71 | 218.3 | -3.68 | 360 | 19.68 | 215.7 | -3.66 |
| 370 | 19.76 | 218.0 | -3.62 | 370 | 19.73 | 215.4 | -3.60 |
| 380 | 19.81 | 217.7 | -3.57 | 380 | 19.78 | 215.3 | -3.55 |
| 390 | 19.85 | 217.5 | -3.52 | 390 | 19.82 | 215.2 | -3.51 |
| 400 | 19.89 | 217.3 | -3.48 | 400 | 19.86 | 215.3 | -3.47 |
| 410 | 19.93 | 217.1 | -3.44 | 410 | 19.90 | 215.3 | -3.43 |
| 420 | 19.96 | 217.0 | -3.40 | 420 | 19.93 | 215.4 | -3.40 |
| 430 | 19.99 | 216.9 | -3.37 | 430 | 19.96 | 215.5 | -3.37 |
| 440 | 20.01 | 216.7 | -3.35 | 440 | 19.99 | 215.6 | -3.35 |
| 450 | 20.04 | 216.5 | -3.31 | 450 | 20.01 | 215.7 | -3.33 |
| 460 | 20.05 | 216.3 | -3.30 | 460 | 20.03 | 215.8 | -3.31 |
| 470 | 20.07 | 216.0 | -3.27 | 470 | 20.05 | 215.9 | -3.29 |
| 480 | 20.09 | 215.7 | -3.24 | 480 | 20.06 | 215.9 | -3.28 |
| 490 | 20.10 | 215.3 | -3.23 | 490 | 20.08 | 216.0 | -3.26 |
| 500 | 20.11 | 214.9 | -3.21 | 500 | 20.09 | 216.1 | -3.26 |
| 510 | 20.12 | 214.5 | -3.19 | 510 | 20.09 | 216.2 | -3.26 |
| 520 | 20.12 | 214.1 | -3.19 | 520 | 20.10 | 216.2 | -3.25 |
| 530 | 20.13 | 213.8 | -3.17 | 530 | 20.10 | 216.4 | -3.25 |
| 540 | 20.13 | 213.6 | -3.17 | 540 | 20.11 | 216.5 | -3.24 |
| 550 | 20.14 | 213.6 | -3.16 | 550 | 20.11 | 216.7 | -3.25 |

Remark: brown colour letters show not consistent data!

TWO ANTENNAS VERTICALLY STACKED AT 550CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21608XL3D9 | No loss | 20.23 | 27.70 | 20.34 | 6.64 | 27.8 | 9.6 | 211.7 | -3.03 | 143.100 - 146.050 |
|  | Aluminum | 20.14 | 27.58 | 20.34 | 6.66 | 27.8 | 9.6 | 213.6 | -3.16 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 550+550+550 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21608XL3D9 | No losss | 23.26 | 27.93 | 20.21 | 8.27 | 28.0 | 3.8 | 210.8 | +0.02 | 143.100 - 146.050 |
|  | Aluminum | 23.17 | 27.80 | 20.22 | 8.28 | 28.0 | 3.8 | 211.6 | -0.11 |  |

FOUR ANTENNAS IN H- STACK, H=520 CM, V=550 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21608XL3D9 | No losss | 23.26 | 28.04 | 7.74 | 6.58 | 10.4 | 9.8 | 211.1 | +0.01 | 143.100 - 146.050 |
|  | Aluminum | 23.17 | 27.91 | 7.75 | 6.59 | 10.4 | 9.8 | 213.0 | -0.11 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE2 from 890 to 900 mm

- Change length of D3 from 924 to 922 mm

- Change length of D4 from 904 to 902 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No loss | 17.15 | 27.98 | 20.66 | 17.61 | 27.6 | 28.4 | 216.0 | -6.20 | 143.100 - 146.050 |
|  |  | Aluminum | 17.06 | 27.86 | 20.63 | 17.61 | 27.6 | 28.4 | 217.8 | -6.32 |  |
|  | Vertical part | No losss | 17.26 | 29.05 | 15.37 | 18.75 | 27.6 | 26.8 | 221.6 | -6.20 | 142.900 – 146.200 |
|  |  | Aluminum | 17.18 | 28.91 | 15.39 | 18.77 | 27.4 | 26.8 | 223.2 | -6.31 |  |