ANTENNA Y21810XL3D11

Good stacking characteristics at any vertical and horizontal distances

| PERFORMANCES: (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| QY21810XL3D11 | 17 | 11985 | 17.73 | 45.23 | 20.39 | 17.28 | 26.0 | 26.6 | 214.7 | -5.59 | 4100 |

* ΔF given for SWR=1,5

| PERFORMANCES: (Loss included) |  |  |  |
| --- | --- | --- | --- |
| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| Aluminum rods | 17.66 | 216.1 | -5.69 |

| DIMENSIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref | De1 | De2 | D1 | D2 | D3 | D4 | D5 | DQ1 | DQ2 | DQ3 | DQ4 | DQ5 | DQ6 | DQ7 | DQ8 | DQ9 | DQ10 | DQ11 |
| Pos | 0 | 190 | 325 | 400 | 700 | 1255 | 1936 | 2718 | 3539 | 4358 | 5237 | 6108 | 6965 | 7865 | 8689 | 9535 | 10395 | 11275 | 11985 |
| Length | 1008 | 1008 | 910 | 932 | 942 | 916 | 896 | 878 | 522 | 508 | 492 | 488 | 478 | 472 | 466 | 458 | 450 | 444 | 438 |
| REMARKS : All elements made of Al rods Ǿ10mm The height of the rectangles 500mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Building hints: No boom correction included.

| COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| QY21810XL3D11 | 11985 | No loss | 17.73 | 214.7 | -5.59 | 142.100 – 146.200 |
|  |  | Aluminum | 17.66 | 216.1 | -5.69 |  |
| EF0217-5X | 11940 | No loss | 17.66 | 217.1 | -5.71 | 141.500 - 145.600 |
|  |  | Aluminum | 17.51 | 220.0 | -5.91 |  |

DIAGRAMS (no loss included)

DIAGRAMS (loss included)

STACKING

| QY21810XL3D11 - STACKING TABLE (including loss for aluminum – AGT = -0.07 dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VERTICAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) | HORIZONTAL DISTANCE (cm) | G (dB) | TA (°K) | G/T (dB) |
| 100 | 17.52 | 225.2 | -6.01 | 100 |  |  |  |
| 110 | 17.89 | 221.0 | -5.55 | 110 |  |  |  |
| 120 | 18.15 | 219.3 | -5.26 | 120 | 17.50 | 237.2 | -6.25 |
| 130 | 18.35 | 219.0 | -5.05 | 130 | 17.96 | 230.9 | -5.67 |
| 140 | 18.51 | 218.5 | -4.89 | 140 | 18.27 | 226.6 | -5.28 |
| 150 | 18.65 | 217.0 | -4.71 | 150 | 18.49 | 224.0 | -5.01 |
| 160 | 18.77 | 215.2 | -4.56 | 160 | 18.66 | 222.3 | -4.81 |
| 170 | 18.89 | 214.1 | -4.42 | 170 | 18.79 | 221.4 | -4.66 |
| 180 | 18.98 | 213.7 | -4.32 | 180 | 18.90 | 220.9 | -4.54 |
| 190 | 19.07 | 213.6 | -4.23 | 190 | 19.00 | 220.7 | -4.44 |
| 200 | 19.16 | 213.8 | -4.14 | 200 | 19.09 | 220.6 | -4.35 |
| 210 | 19.24 | 214.0 | -4.07 | 210 | 19.18 | 220.6 | 4.26 |
| 220 | 19.31 | 214.4 | -4.00 | 220 | 19.25 | 220.7 | -4.19 |
| 230 | 19.39 | 214.9 | -3.93 | 230 | 19.32 | 220.8 | -4.12 |
| 240 | 19.45 | 215.4 | -3.88 | 240 | 19.39 | 220.8 | -4.05 |
| 250 | 19.52 | 216.1 | -3.83 | 250 | 19.46 | 220.7 | -3.98 |
| 260 | 19.58 | 216.8 | -3.78 | 260 | 19.53 | 220.5 | -3.90 |
| 270 | 19.64 | 217.5 | -3.73 | 270 | 19.59 | 220.2 | -3.84 |
| 280 | 19.70 | 218.2 | -3.69 | 280 | 19.66 | 219.8 | -3.76 |
| 290 | 19.76 | 218.7 | -3.64 | 290 | 19.72 | 219.2 | -3.69 |
| 300 | 19.82 | 219.1 | -3.59 | 300 | 19.79 | 218.6 | -3.61 |
| 310 | 19.88 | 219.2 | -3.53 | 310 | 19.85 | 217.9 | -3.53 |
| 320 | 19.93 | 219.1 | -3.48 | 320 | 19.91 | 217.3 | -3.46 |
| 330 | 19.99 | 219.0 | -3.41 | 330 | 19.97 | 216.6 | -3.39 |
| 340 | 20.05 | 218.8 | -3.35 | 340 | 20.03 | 216.0 | -3.31 |
| 350 | 20.11 | 218.5 | -3.29 | 350 | 20.09 | 215.5 | -3.24 |
| 360 | 20.16 | 218.3 | -3.23 | 360 | 20.15 | 215.0 | -3.17 |
| 370 | 20.22 | 218.0 | -3.17 | 370 | 20.20 | 214.7 | -3.12 |
| 380 | 20.27 | 217.7 | -3.11 | 380 | 20.26 | 214.4 | -3.05 |
| 390 | 20.32 | 217.2 | -3.05 | 390 | 20.30 | 214.3 | -3.01 |
| 400 | 20.36 | 216.9 | -3.00 | 400 | 20.35 | 214.2 | -2.96 |
| 410 | 20.41 | 216.5 | -2.94 | 410 | 20.39 | 214.1 | -2.92 |
| 420 | 20.45 | 216.1 | -2.90 | 420 | 20.44 | 214.1 | -2.87 |
| 430 | 20.49 | 215.8 | -2.85 | 430 | 20.47 | 214.1 | -2.84 |
| 440 | 20.52 | 215.4 | -2.81 | 440 | 20.51 | 214.1 | -2.80 |
| 450 | 20.55 | 215.1 | -2.78 | 450 | 20.54 | 214.1 | -2.77 |
| 460 | 20.58 | 214.7 | -2.74 | 460 | 20.57 | 214.2 | -2.74 |
| 470 | 20.61 | 214.3 | -2.70 | 470 | 20.59 | 214.2 | -2.72 |
| 480 | 20.63 | 213.9 | -2.67 | 480 | 20.61 | 214.2 | -2.70 |
| 490 | 20.65 | 213.5 | -2.64 | 490 | 20.63 | 214.2 | -2.68 |
| 500 | 20.67 | 213.1 | -2.62 | 500 | 20.65 | 214.2 | -2.66 |
| 510 | 20.69 | 212.7 | 2.59 | 510 | 20.66 | 214.2 | -2.65 |
| 520 | 20.70 | 212.4 | -2.57 | 520 | 20.68 | 214.2 | -2.63 |
| 530 | 20.71 | 212.1 | -2.55 | 530 | 20.69 | 214.2 | -2.62 |
| 540 | 20.72 | 211.8 | -2.54 | 540 | 20.70 | 214.3 | -2.61 |
| 550 | 20.73 | 211.8 | -2.53 | 550 | 20.70 | 214.4 | -2.61 |
| 560 | 20.73 | 211.8 | -2.53 | 560 | 20.71 | 214.5 | -2.60 |
| 570 | 20.73 | 211.9 | -2.53 | 570 | 20.71 | 214.7 | -2.61 |

Remark: brown colour letters show not consistent data!

TWO ANTENNAS VERTICALLY STACKED AT 550CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 2xQY21810XL3D11 | No losss | 20.80 | 42.97 | 20.71 | 7.63 | 26.2 | 9.6 | 210.3 | -2.43 | 142.100 – 146.200 |
|  | Aluminum | 20.73 | 42.27 | 20.70 | 7.64 | 26.2 | 9.6 | 211.8 | -2.53 |  |

FOUR ANTENNAS VERTICALLY STACKED AT 550+550+550 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21810XL3D11 | No losss | 23.83 | 43.85 | 19.79 | 9.95 | 26.4 | 3.8 | 209.9 | +0.61 | 142.100 – 146.200 |
|  | Aluminum | 23.77 | 43.10 | 19.77 | 9.97 | 26.4 | 3.8 | 211.3 | +0.52 |  |

FOUR ANTENNAS IN H- STACK, H=540 CM, V=550 CM

| Performances |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
| 4xQY21810XL3D11 | No losss | 23.83 | 43.66 | 7.99 | 7.53 | 9.8 | 9.6 | 209.3 | +0.62 | 142.100 – 146.200 |
|  | Aluminum | 23.76 | 42.89 | 8.01 | 7.55 | 9.8 | 9.6 | 210.8 | +0.52 |  |

XPOL VERSION

Horizontal components: Same as above

Vertical components: Same as above, except:

- All vertical yagi elements positioned 20 mm backward regarding horizontals.

- All crossed elements must cross each other in their centerlines

HORIZONTAL PERFORMANCES

Everything as the above data

VERTICAL PERFORMANCES with no corrections

Loss included condition (for Aluminum)

VERTICAL PERFORMANCES after corrections,  loss included condition

If you want to get better SWR diagram for vertical antenna you can do the following:

- Change length of DE2 from 910 to 900 mm

- Change length of D3 from 916 to 914 mm

- Change length of D4 from 896 to 894 mm

- Change position of D2 for +2 mm

In this case the SWR diagram will be like this:

| PERFORMANCES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TYPE | Condition | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | B (MHz) for SWR=1.5 |
|  | Horizontal part | No losss | 17.73 | 45.23 | 20.39 | 17.28 | 26.0 | 26.6 | 214.7 | -5.59 | 142.100 – 146.200 |
|  |  | Aluminum | 17.66 | 44.40 | 20.39 | 17.28 | 26.0 | 26.6 | 216.1 | -5.69 |  |
|  | Vertical part | No losss | 17.88 | 35.61 | 14.55 | 17.59 | 25.4 | 25.0 | 219.5 | -5.53 | 141.400 – 146.400 |
|  |  | Aluminum | 17.82 | 35.96 | 14.56 | 17.60 | 25.4 | 25.0 | 220.8 | -5.62 |  |