| ANTENNA QY70805XL8D3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  |  |  |  | Boomlength (mm) |  |  |  |
| LFA |  |  | 4 |  |  |  |  | 3 |  |  |  | 8 |  |  |  |  |  | 1165 |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  |  | De1 |  | De2 |  |  | D1 |  | D2 |  |  | D3 |  | DQ1 |  |  | DQ2 |  | DQ3 |
| Position | 0 |  |  | 54 |  | 104 |  |  | 141 |  | 254 |  |  | 472 |  | 708 |  |  | 959 |  | 1165 |
| Length | 335 |  |  | 354 |  | 282 |  |  | 309 |  | 306 |  |  | 293 |  | 175 |  |  | 171 |  | 165 |
| Remark | All elements made of Al tubes Ǿ5 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  | F/Sh (dBi) |  | F/Sv (dBi) |  |  | Hor (◦) |  |  |  |  | Ver (◦) |  | Temp (◦K) |  |  | G/T (dB) |  |
| 13.61 |  | 25.89 |  |  | 19.54 |  | 15.67 |  |  | 40.0 |  |  |  |  | 44.0 |  | 39.4 |  |  | -1.96 |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  | G (dBi) |  | Tloss (K) |  |  | TA (◦K) |  |  |  |  | G/T (dB) |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |
| After NEC/EZNEC |  |  |  |  | 13.56 |  | 3.4 |  |  | 39.4 |  |  |  |  | -2.36 |  | 420.800 – 438.500 |  |  |  |  |
| After KF2YN correction |  |  |  |  | 13.50 |  | 3.4 |  |  | 42.4 |  |  |  |  | -2.77 |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |
| Horizontal distance |  |  |  |  | 1140 |  |  |  |  |  |  |  |  |  |  |  | 1140 |  |  |  |  |
| Vertical distance |  |  |  |  |  |  |  |  |  | 880 |  |  |  |  |  |  | 880 |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  | Tloss (K) |  |  |  |  |  | G/T (dB) |  |  |  |
| 880 |  |  | 16.34 |  |  |  |  | 38.7 |  |  |  | 3.4 |  |  |  |  |  | +0.46 |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  | G/T (dB) |  |  |
| 1140 |  |  | 16.54 |  |  |  |  | 42.7 |  |  |  |  | 3.4 |  |  |  |  |  | +0.24 |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  | G/T (dB) |  |  |
| H = 1140 |  |  | 19.42 |  |  |  |  | 39.5 |  |  |  |  | 3.4 |  |  |  |  |  | +3.46 |  |  |
| V = 880 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |