| ANTENNA QY70908XL4D2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  |  |  |  | Boomlength (mm) |  |  |  |  |
| LFA |  |  | 6 |  |  |  |  | 2 |  |  |  | 9 |  |  |  |  |  | 1475 |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  | 50 |  | 110 |  | 155 |  | 274 |  | 474 |  |  |  | 719 | 978 |  |  |  | 1246 |  | 1475 |
| Position | 331 |  | 330 |  | 330 |  | 303 |  | 301 |  | 290 |  |  |  | 283 | 274.5 |  |  |  | 171 |  | 157 |
| Length | 0 |  | 50 |  | 110 |  | 154 |  | 277 |  | 481 |  |  |  | 719 | 981 |  |  |  | 1249 |  | 1475 |
| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 165 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) |  | F/Sv (dBi) |  |  |  | Hor (◦) |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  | G/T (dB) |  |
| 14.25 |  | 23.20 |  | 20.29 |  | 17.42 |  |  |  | 37.2 |  |  |  | 41.0 |  |  | 35.0 |  |  |  | -0.88 |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) |  | Tloss (K) |  |  |  | TA (◦K) |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |
| After NEC/EZNEC |  |  |  | 14.21 |  | 2.6 |  |  |  | 35.0 |  |  |  | -1.23 |  |  | 421.500 – 438.000 |  |  |  |  |  |
| After KF2YN correction |  |  |  | 14.21 |  | 2.6 |  |  |  | 37.3 |  |  |  | -1.50 |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  | 1280 |  |  |  |  |  |  | 1280 |  |  |  |  |  |
| Vertical distance |  |  |  | 940 |  |  |  |  |  |  |  |  |  |  |  |  | 940 |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  | Tloss (K) |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 940 |  |  | 17.04 |  |  |  |  | 34.9 |  |  |  | 2.6 |  |  |  |  |  | +1.61 |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  | G/T (dB) |  |  |  |
| 1280 |  |  | 17.22 |  |  |  |  | 37.8 |  |  |  |  | 2.6 |  |  |  |  |  | +1.45 |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  | G/T (dB) |  |  |  |
| H = 1280 |  |  | 20.11 |  |  |  |  | 36.1 |  |  |  |  | 2.6 |  |  |  |  |  | +3.54 |  |  |  |
| V = 940 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |