| ANTENNA QY71006XL1D4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  | Number of yagi elements |  |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  |  |  |  | Boomlength (mm) |  |  |  |  |  |
| LFA |  |  |  | 5 |  |  |  |  | 4 |  |  |  | 10 |  |  |  |  |  | 1665 |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  |  | De2 | D1 |  |  | D2 |  | D3 |  |  |  | D4 | DQ1 |  |  | DQ2 |  |  | DQ3 | DQ4 |
| Position | 0 |  | 50 |  |  | 100 | 129 |  |  | 220 |  | 402 |  |  |  | 632 | 882 |  |  | 1139 |  |  | 1435 | 1665 |
| Length | 338 |  | 335 |  |  | 335 | 308 |  |  | 309.5 |  | 298.5 |  |  |  | 288.5 | 173 |  |  | 163 |  |  | 161 | 153 |
| Remark | All elements made of Al tubes Ǿ6 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  | Hor (◦) |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  | G/T (dB) |  |  |
| 14.63 |  | 29.05 |  |  | 22.75 |  |  | 19.81 |  |  | 36.6 |  |  |  | 39.2 |  |  | 30.4 |  |  |  | +0.32 |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  | TA (◦K) |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  | 14.57 |  |  | 4.0 |  |  | 30.4 |  |  |  | -0.29 |  |  | 424.000 – 438.000 |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  | 14.51 |  |  | 4.0 |  |  | 34.0 |  |  |  | -0.80 |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  | 1320 |  |  |  |  |  |  | 1320 |  |  |  |  |  |  |
| Vertical distance |  |  |  |  | 980 |  |  |  |  |  |  |  |  |  |  |  |  | 980 |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  | Tloss (K) |  |  |  |  |  | G/T (dB) |  |  |  |  |  |
| 980 |  |  |  | 17.35 |  |  |  |  | 32.7 |  |  |  | 4.0 |  |  |  |  |  | +2.21 |  |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |
| 1320 |  |  |  | 17.55 |  |  |  |  | 34.1 |  |  |  |  | 4.0 |  |  |  |  |  |  | +2.23 |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |
| H = 1320 |  |  |  | 20.42 |  |  |  |  | 33.3 |  |  |  |  | 4.0 |  |  |  |  |  |  | +5.20 |  |  |  |
| V = 980 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |