| ANTENNA QY71105XL2D5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  | Number of yagi elements |  |  |  |  | Number of oblong elements |  |  |  |  | Total number of elements |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |
| LFA |  |  |  | 5 |  |  |  |  | 5 |  |  |  |  | 11 |  |  |  |  |  |  | 2005 |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  |  | De2 | D1 |  |  | D2 | D3 |  | D4 |  |  |  | DQ1 | DQ2 |  | DQ3 |  |  | DQ4 |  | DQ5 |
| Position | 0 |  | 50 |  |  | 105 | 138 |  |  | 231 | 415 |  | 646 |  |  |  | 914 | 1174 |  | 1466 |  |  | 1764 |  | 2005 |
| Length | 338 |  | 334 |  |  | 334 | 311.5 |  |  | 311.5 | 299.5 |  | 289 |  |  |  | 173 | 165 |  | 159 |  |  | 155 |  | 151 |
| Remark | All elements made of Al tubes Ǿ5 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  | Hor (◦) |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  |  | G/T (dB) |  |
| 15.22 |  | 27.76 |  |  | 22.23 |  |  | 17.96 |  |  |  | 34.6 |  |  |  | 36.8 |  |  | 28.5 |  |  |  |  | +1.25 |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  | TA (◦K) |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  | 15.16 |  |  | 4.6 |  |  |  | 28.5 |  |  |  | +0.52 |  |  | 422.000 – 439.100 |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  | 15.09 |  |  | 4.6 |  |  |  | 32.6 |  |  |  | -0.05 |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  | 1320 |  |  |  |  |  |  | 1320 |  |  |  |  |  |  |
| Vertical distance |  |  |  |  | 1140 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1140 |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 1140 |  |  |  | 18.03 |  |  |  |  | 32.8 |  |  |  |  | 4.6 |  |  |  |  |  |  | +2.87 |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |
| 1320 |  |  |  | 18.11 |  |  |  |  | 32.4 |  |  |  |  |  | 4.6 |  |  |  |  |  |  | +3.00 |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |
| H = 1320 |  |  |  | 21.09 |  |  |  |  | 33.3 |  |  |  |  |  | 4.6 |  |  |  |  |  |  | +5.86 |  |  |  |
| V = 1140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |