| ANTENNA QY71110XL4D5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  | Number of yagi elements |  |  |  |  | Number of oblong elements |  |  |  |  | Total number of elements |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |
| LFA |  |  |  | 5 |  |  |  |  | 5 |  |  |  |  | 11 |  |  |  |  |  |  | 2005 |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  |  | De2 | D1 |  |  | D2 | D3 |  | D4 |  |  |  | DQ1 | DQ2 |  | DQ3 |  |  | DQ4 |  | DQ5 |
| Position | 0 |  | 60 |  |  | 100 | 131 |  |  | 222 | 411 |  | 644 |  |  |  | 896 | 1168 |  | 1461 |  |  | 1772 |  | 2005 |
| Length | 333 |  | 337 |  |  | 337 | 296 |  |  | 301 | 289.5 |  | 278.5 |  |  |  | 171 | 159 |  | 155 |  |  | 153 |  | 151 |
| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  | Hor (◦) |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  |  | G/T (dB) |  |
| 15.24 |  | 25.25 |  |  | 23.84 |  |  | 19.32 |  |  |  | 34.4 |  |  |  | 36.4 |  |  | 29.0 |  |  |  |  | +1.14 |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  | TA (◦K) |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  | 15.20 |  |  | 2.3 |  |  |  | 29.0 |  |  |  | +0.72 |  |  | 424.000 – 438.500 |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  | 14.15 |  |  | 2.3 |  |  |  | 31.1 |  |  |  | +0.23 |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  | 1320 |  |  |  |  |  |  | 1320 |  |  |  |  |  |  |
| Vertical distance |  |  |  |  | 1120 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1120 |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 1120 |  |  |  | 18.06 |  |  |  |  | 30.7 |  |  |  |  | 2.3 |  |  |  |  |  |  | +3.20 |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |
| 1320 |  |  |  | 18.16 |  |  |  |  | 30.9 |  |  |  |  |  | 2.3 |  |  |  |  |  |  | +3.27 |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |
| H = 1320 |  |  |  | 21.08 |  |  |  |  | 31.2 |  |  |  |  |  | 2.3 |  |  |  |  |  |  | +6.15 |  |  |  |
| V = 1120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |