| ANTENNA QY71306XL8D6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  | Number of yagi elements |  |  |  |  |  | Number of oblong elements |  |  |  |  | Total number of elements |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |  |
| LFA |  |  |  | 6 |  |  |  |  |  | 6 |  |  |  |  | 13 |  |  |  |  |  |  | 2645 |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  | De2 |  | D1 | D2 |  |  | D3 | D4 |  | D5 |  |  | DQ1 |  | DQ2 | DQ3 |  |  |  | DQ4 |  | DQ5 | DQ6 |
| Position | 0 |  | 71 |  | 105 |  | 136 | 235 |  |  | 417 | 645 |  | 907 |  |  | 1192 |  | 1472 | 1776 |  |  |  | 2078 |  | 2394 | 2645 |
| Length | 334 |  | 341 |  | 330 |  | 303.5 | 306.5 |  |  | 297 | 289.5 |  | 281 |  |  | 167 |  | 159 | 153 |  |  |  | 151 |  | 149 | 147 |
| Remark | All elements made of Al tubes Ǿ6 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  | Hor (◦) |  |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  | G/T (dB) |  |  |
| 16.10 |  | 30.49 |  |  |  | 22.05 |  |  | 18.82 |  |  |  | 31.6 |  |  |  |  | 33.2 |  |  | 26.7 |  |  |  | +2.39 |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  | TA (◦K) |  |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  |  | 16.06 |  |  | 3.4 |  |  |  | 26.7 |  |  |  |  | +1.77 |  |  | 416.000 – 439.800 |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  |  | 16.00 |  |  | 3.4 |  |  |  | 29.8 |  |  |  |  | +1.25 |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  |  | 1340 |  |  |  |  |  |  |  | 1340 |  |  |  |  |  |  |
| Vertical distance |  |  |  |  |  | 1580 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1580 |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |  |  |
| 1580 |  |  |  | 19.08 |  |  |  |  |  | 29.2 |  |  |  |  | 3.4 |  |  |  |  |  |  | +4.42 |  |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 1340 |  |  |  | 18.94 |  |  |  |  |  | 29.1 |  |  |  |  |  | 3.4 |  |  |  |  |  |  | +4.29 |  |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| H = 1340 |  |  |  | 22.03 |  |  |  |  |  | 29.2 |  |  |  |  |  | 3.4 |  |  |  |  |  |  | +7.37 |  |  |  |  |
| V = 1580 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |