| ANTENNA QY71510XL3D8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  | Number of yagi elements |  |  |  |  |  |  | Number of oblong elements |  |  |  |  | Total number of elements |  |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |  |
| LFA |  |  |  | 6 |  |  |  |  |  |  | 8 |  |  |  |  | 15 |  |  |  |  |  |  |  | 3225 |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  | De2 |  | D1 | D2 |  | D3 |  | D4 | D5 |  | DQ1 |  |  | DQ2 |  | DQ3 | DQ4 |  | DQ5 |  |  | DQ6 |  | DQ7 | DQ8 |
| Position | 0 |  | 54 |  | 91 |  | 134 | 235 |  | 424 |  | 654 | 916 |  | 1195 |  |  | 1466 |  | 1791 | 2076 |  | 2392 |  |  | 2692 |  | 2986 | 3225 |
| Length | 331 |  | 342 |  | 342 |  | 302 | 300.5 |  | 289 |  | 281 | 272 |  | 168 |  |  | 158 |  | 153 | 151 |  | 149 |  |  | 147 |  | 145 | 144 |
| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  |  | Hor (◦) |  |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  |  | G/T (dB) |  |  |
| 16.91 |  | 34.95 |  |  |  | 21.96 |  |  | 18.23 |  |  |  |  | 28.8 |  |  |  |  | 30.0 |  |  | 26.8 |  |  |  |  | +3.08 |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  |  | TA (◦K) |  |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  |  | 16.87 |  |  | 2.9 |  |  |  |  | 26.8 |  |  |  |  | +2.59 |  |  | 423.000 – 439.000 |  |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  |  | 16.82 |  |  | 2.9 |  |  |  |  | 29.4 |  |  |  |  | +2.14 |  |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  |  |  | 1760 |  |  |  |  |  |  |  | 1760 |  |  |  |  |  |  |  |
| Vertical distance |  |  |  |  |  | 1780 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1780 |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  | TA (K) |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |  |
| 1780 |  |  |  | 19.91 |  |  |  |  |  |  | 27.8 |  |  |  |  | 2.9 |  |  |  |  |  |  |  | +5.47 |  |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 1760 |  |  |  | 19.88 |  |  |  |  |  |  | 29.0 |  |  |  |  |  | 2.9 |  |  |  |  |  |  |  | +5.26 |  |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| H = 1760 |  |  |  | 22.96 |  |  |  |  |  |  | 27.6 |  |  |  |  |  | 2.9 |  |  |  |  |  |  |  | +8.55 |  |  |  |  |
| V = 1780 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |