| ANTENNA QY71710XL7D10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  | Number of yagi elements |  |  | Number of oblong elements |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  | 6 |  |  | 10 |  |  | 17 |  |  | 3825 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 | De2 |  | D1 | D2 |  | D3 | D4 |  | D5 | DQ1 |
| Position | 0 |  | 45 | 95 |  | 138 | 241 |  | 430 | 660 |  | 920 | 1196 |
| Length | 331 |  | 338 | 338 |  | 301 | 300 |  | 289 | 281.5 |  | 273.5 | 170 |
|  | DQ2 |  | DQ3 | DQ4 |  | DQ5 | DQ6 |  | DQ7 | DQ8 |  | DQ9 | DQ10 |
| Position | 1461 |  | 1772 | 2064 |  | 2368 | 2673 |  | 2972 | 3292 |  | 3578 | 3825 |
| Length | 162 |  | 157 | 154 |  | 151 | 147 |  | 143 | 141 |  | 139 | 137 |

| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 17.50 |  | 38.68 |  | 22.62 | 19.05 |  | 27.0 |  |  | 27.6 | 25.8 |  |  | +3.84 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 17.46 | 2.9 |  | 25.8 |  |  | +3.31 | 420.000 – 439.000 |  |  |  |
| After KF2YN correction |  |  |  | 17.41 | 2.9 |  | 28.4 |  |  | +2.88 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 1780 |  |  |  | 1780 |  |  |  |
| Vertical distance |  |  |  | 1800 |  |  |  |  |  |  | 1800 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1800 |  |  | 20.47 |  |  | 27.1 |  | 2.9 |  |  |  | +6.14 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 1780 |  |  | 20.45 |  |  | 27.7 |  |  | 2.9 |  |  |  | +6.03 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 1780 |  |  | 23.52 |  |  | 26.6 |  |  | 2.9 |  |  |  | +9.27 |  |
| V = 1800 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |