| ANTENNA QY71806XL6D11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  | 11 |  |  | 18 |  |  | 4125 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 |  | D4 | D5 |  | DQ1 | DQ2 |
| Position | 0 | 65 |  | 100 | 131 |  | 233 | 419 |  | 648 | 905 |  | 1182 | 1456 |
| Length | 336 | 349 |  | 304 | 304 |  | 307.5 | 298 |  | 291.5 | 283.5 |  | 171 | 163 |
|  | DQ3 | DQ4 |  | DQ5 | DQ6 |  | DQ7 | DQ8 |  | DQ9 | DQ10 |  | DQ11 |  |
| Position | 1754 | 2058 |  | 2364 | 2672 |  | 2974 | 3276 |  | 3594 | 3878 |  | 4125 |  |
| Length | 159 | 155 |  | 151 | 149 |  | 145 | 143 |  | 141 | 139 |  | 137 |  |

| Remark | All elements made of Al tubes Ǿ6 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 17.66 |  | 45.55 |  | 23.82 | 19.56 |  | 26.8 |  |  | 27.4 | 24.9 |  |  | +4.57 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 17.60 | 4.3 |  | 24.9 |  |  | +3.71 | 420.000 – 438.500 |  |  |  |
| After KF2YN correction |  |  |  | 17.52 | 4.3 |  | 28.8 |  |  | +2.93 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 1840 |  |  |  | 1840 |  |  |  |
| Vertical distance |  |  |  | 1880 |  |  |  |  |  |  | 1880 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1880 |  |  | 20.59 |  |  | 27.7 |  | 4.3 |  |  |  | +6.16 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 1840 |  |  | 20.56 |  |  | 28.3 |  |  | 4.3 |  |  |  | +6.04 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 1840 |  |  | 23.64 |  |  | 27.4 |  |  | 4.3 |  |  |  | +9.26 |  |
| V = 1880 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |