| ANTENNA QY71810XL7D11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  | 11 |  |  | 18 |  |  | 4125 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 |  | D4 | D5 |  | DQ1 | DQ2 |
| Position | 0 | 50 |  | 95 | 136 |  | 234 | 428 |  | 658 | 918 |  | 1194 | 1462 |
| Length | 331 | 334 |  | 334 | 302 |  | 301 | 289.5 |  | 281 | 273.5 |  | 170 | 162 |
|  | DQ3 | DQ4 |  | DQ5 | DQ6 |  | DQ7 | DQ8 |  | DQ9 | DQ10 |  | DQ11 |  |
| Position | 1772 | 2066 |  | 2368 | 2673 |  | 2972 | 3292 |  | 3578 | 3876 |  | 4125 |  |
| Length | 157 | 153 |  | 151 | 147 |  | 143 | 141 |  | 141 | 139 |  | 137 |  |

| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 17.75 |  | 29.06 |  | 22.53 | 18.72 |  | 26.2 |  |  | 26.8 | 25.6 |  |  | +4.07 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 17.71 | 3.2 |  | 25.6 |  |  | +3.52 | 426.500 – 438.500 |  |  |  |
| After KF2YN correction |  |  |  | 17.67 | 3.2 |  | 28.5 |  |  | +3.12 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 1840 |  |  |  | 1840 |  |  |  |
| Vertical distance |  |  |  | 1900 |  |  |  |  |  |  | 1900 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1900 |  |  | 20.75 |  |  | 27.1 |  | 3.2 |  |  |  | +6.42 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 1840 |  |  | 20.72 |  |  | 27.7 |  |  | 3.2 |  |  |  | +6.29 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 1840 |  |  | 23.79 |  |  | 26.8 |  |  | 3.2 |  |  |  | +9.51 |  |
| V = 1940 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |