| ANTENNA QY72108XL4D14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  | 14 |  |  |  | 21 |  |  | 5025 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 | D4 |  | D5 | DQ1 |  | DQ2 | DQ3 |
| Position | 0 | 66 |  | 98 | 133 |  | 232 | 422 | 654 |  | 921 | 1185 |  | 1474 | 1774 |
| Length | 333 | 348 |  | 300 | 303 |  | 305 | 294.5 | 291.5 |  | 279 | 171 |  | 162 | 159 |
|  | DQ4 | DQ5 |  | DQ6 | DQ7 |  | DQ8 | DQ9 | DQ10 |  | DQ11 | DQ12 |  | DQ13 | DQ13 |
| Position | 2062 | 2388 |  | 2682 | 2982 |  | 3286 | 3580 | 3888 |  | 4178 | 4482 |  | 4772 | 5025 |
| Length | 155 | 151 |  | 151 | 147 |  | 143 | 143 | 137 |  | 137 | 137 |  | 137 | 131 |

| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 18.40 |  | 40.88 |  | 23.07 | 19.20 |  | 24.8 |  |  | 25.2 | 24.2 |  |  | +5.71 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 18.24 | 2.9 |  | 24.2 |  |  | +4.85 | 426.500 – 437.600 |  |  |  |
| After KF2YN correction |  |  |  | 18.28 | 2.9 |  | 25.8 |  |  | +4.16 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 1900 |  |  |  | 1900 |  |  |  |
| Vertical distance |  |  |  | 1900 |  |  |  |  |  |  | 1900 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1900 |  |  | 21.32 |  |  | 25.8 |  | 2.9 |  |  |  | +7.20 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 1900 |  |  | 21.31 |  |  | 26.3 |  |  | 2.9 |  |  |  | +7.11 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 1900 |  |  | 24.36 |  |  | 25.6 |  |  | 2.9 |  |  |  | +10.28 |  |
| V = 1900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |