| ANTENNA QY72310XL7D16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  | 16 |  |  |  | 23 |  |  |  | 5595 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 | D4 |  | D5 | DQ1 | DQ2 |  | DQ3 | DQ4 |
| Position | 0 | 70 |  | 120 | 152 |  | 246 | 435 | 672 |  | 932 | 1203 | 1478 |  | 1775 | 2086 |
| Length | 331 | 330 |  | 330 | 295 |  | 301 | 289.5 | 281.5 |  | 274 | 171 | 163 |  | 157 | 155 |
|  | DQ5 | DQ6 |  | DQ7 | DQ8 |  | DQ9 | DQ10 | DQ11 |  | DQ12 | DQ13 | DQ14 |  | DQ15 | DQ16 |
| Position | 2378 | 2678 |  | 2972 | 3262 |  | 3556 | 3852 | 4148 |  | 4454 | 4752 | 5058 |  | 5346 | 5595 |
| Length | 153 | 151 |  | 147 | 143 |  | 139 | 135 | 133 |  | 131 | 131 | 131 |  | 129 | 123 |

| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 18.65 |  | 35.34 |  | 24.47 | 20.34 |  | 24.2 |  |  | 24.6 | 24.0 |  |  | +5.44 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 18.61 | 3.2 |  | 24.0 |  |  | +4.85 | 425.500 – 437.800 |  |  |  |
| After KF2YN correction |  |  |  | 18.55 | 3.2 |  | 26.9 |  |  | +4.25 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2020 |  |  |  | 2020 |  |  |  |
| Vertical distance |  |  |  | 1900 |  |  |  |  |  |  | 1900 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1900 |  |  | 21.56 |  |  | 26.3 |  | 3.2 |  |  |  | +7.36 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2020 |  |  | 21.58 |  |  | 26.7 |  |  | 3.2 |  |  |  | +7.32 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2020 |  |  | 24.60 |  |  | 26.0 |  |  | 3.2 |  |  |  | +10.45 |  |
| V = 1900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |