| ANTENNA QY72710XL9D20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  |  | Boomlength (mm) |  |  |  |
| LFA |  |  | 6 |  |  |  | 20 |  |  |  | 27 |  |  |  | 6825 |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 | D2 |  | D3 | D4 | D5 |  | DQ1 | DQ2 | DQ3 |  | DQ4 | DQ5 | DQ6 |
| Position | 0 | 70 |  | 120 | 152 | 246 |  | 435 | 672 | 932 |  | 1203 | 1478 | 1775 |  | 2086 | 2378 | 2678 |
| Length | 331 | 327 |  | 327 | 295 | 301.5 |  | 289.5 | 282 | 274.5 |  | 171 | 163 | 157 |  | 157 | 155 | 153 |
|  | DQ7 | DQ8 |  | DQ9 | DQ10 | DQ11 |  | DQ12 | DQ13 | DQ14 |  | DQ15 | DQ16 | DQ17 |  | DQ18 | DQ19 | DQ20 |
| Position | 2972 | 3262 |  | 3556 | 3852 | 4148 |  | 4454 | 4752 | 5068 |  | 5356 | 5666 | 5976 |  | 6272 | 6578 | 6825 |
| Length | 151 | 149 |  | 147 | 145 | 143 |  | 141 | 139 | 137 |  | 135 | 131 | 127 |  | 127 | 127 | 123 |

| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 19.55 |  | 31.17 |  | 20.75 | 18.24 |  | 21.6 |  |  | 21.8 | 23.8 |  |  | +6.42 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 19.51 | 3.2 |  | 23.8 |  |  | +5.76 | 426.600 – 437.500 |  |  |  |
| After KF2YN correction |  |  |  | 19.45 | 3.2 |  | 26.7 |  |  | +5.19 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2560 |  |  |  | 2560 |  |  |  |
| Vertical distance |  |  |  | 1900 |  |  |  |  |  |  | 1900 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1900 |  |  | 22.37 |  |  | 26.10 |  | 3.2 |  |  |  | +8.20 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2560 |  |  | 22.52 |  |  | 26.5 |  |  | 3.2 |  |  |  | +8.29 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2560 |  |  | 25.40 |  |  | 25.2 |  |  | 3.2 |  |  |  | +11.22 |  |
| V = 1900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |